Response from TC 129: Glass in building to the

Replies to Mr. G. Katsarakis’s E-mail to the EGF on

Reaction to Fire Classification of Glass Products

The Technical Committee expresses their gratitude to the EGF members for their considered comments on our proposal.

We have examined each comment and our responses on a topic by topic basis are as follows:

- Common test method

We agree with the comments that all construction products must be classified according to an agreed ‘harmonised’ test method, e.g. EN 13501-1 for reaction to fire

However, in the case of EN 13501-5 this is not the case. When initially published it included three now increased to four different test methods that were all based on national/regional tests. The reason being there was no technical agreement on the basic reason/criteria for the test and the subsequent product classification.

Our contention is that the basis of EN 13501-1 with respect to the determination of the ‘calorific value’ of components might have been made without considerations for construction products such as insulating glass units and laminated glass/laminated safety glass. Historically these products have been “classified” at the highest level and regarded as any other glass product. Therefore glass products were probably never considered using the ‘calorific value’ criteria. Evidence that they have contributed to fires has been impossible to obtain. These glass products are then subjected to other test methods, e.g. SBI test, that would require numerous tests due to many different product make-ups, the many and various glazing systems and frame materials used with these products.

- Allowance of NPD rather than Euroclass F

We still believe that EN 13501-1 classification listing should be altered to allow a manufacturer to claim NPD. This will only assist in a few member states but it would remove the present difficulty when a manufacturer does not wish to or does not need to declare a classification for ‘Safety in Case of Fire - Reaction to Fire.

- Guidance Paper G

Comments have been made with respect to Guidance Paper G and we are of course willing to re-examine this document. However, previously its relevance to our particular situation has been unclear. Our problem being that the family laminated glass/laminated safety glass covers a very wide range of thicknesses and make-ups with respect to both glass and organic components.

Any assistance from the EGF on how the Guidance paper can be applied in our case would be most welcome.

- Modification to Member States Regulations

This proposal appears to TC 129 to be the most logical solution to our difficulties. If this approach produced a limited number of ‘Reaction to Fire classifications’ in a consistent manner across the member states then the requirements for testing product families to meet these classes, especially if CWFT was acceptable, might be a solution for the industry.

- Separation of Insulating Glass Units from Laminated Glass/Laminated Safety Glass

From some of the comments this appears to be a sensible way forward. However, considerable detail would be required with respect to unit make-up, unit size/aspect ratio, sealant materials and glass components before such a “deemed to satisfy” document could be finalised.

- Laminated Glass/Laminated Safety Glass

We would appreciate consideration being given to separating out specific product types from this family. The major volume products are the simple ‘safety glasses’, i.e. those made-up from 0,38mm or 0,76mm thick folio interlayers, and these are used in doors, walls, etc. where regulations relating to safety due to accidental human impact is of prime consideration. If this separation allowed a “deemed to satisfy” declaration for the simple products it would be of considerable assistance.

If this was also linked to consistent Member State requirements (see point 4) a manufacturer could find it easier to declare the performance of their products.

This will be of greater importance when under the CPR the manufacturer is required to supply a ‘declaration of performance’.

On behalf of TC129 we hope that the responses above will be of assistance to the EGF with their deliberations on this topic.

We remain available for further discussions on this matter.

Signed on behalf of TC129 by J. Brian Waldron

J. Brian Waldron

Chairman CEN TC129: Glass in building

03/10/2011

EN 13501 – 1: Fire classification of construction products and building elements – Part 1: Classification using test data from reaction to fire tests

EN 13501 – 5: Fire classification of construction products and building elements – Part 5: Classification using data from external fire exposure roof tests.

Guidance paper G: The European classification system for the reaction to Fire Performance of Construction Products.