REGULATION

OF THE COUNCIL OF MINISTERS

of 2 January 2007

on the requirements of the relating content of the natural radioactive isotopes of potassium K-40, the radium Ra-226 and the thorium Th-228 in raw materials and materials applied in buildings intended for the stay of people and live-stock, and also on the content of these isotopes in industrial wastes applied in the building, and on the control of thereof

On the basis of article 6 clause 3 of the Law of  29 November 2000 – The Atomic Law (Journal of Laws of 2004, No. 161, item 1689 as amended ) the following is administered:

§ 1. The Regulation defines the requirements relating to:

- the content of the natural radioactive isotopes of potassium K-40, the radium Ra-226 and the thorium Th-228 in raw materials and in materials applied in buildings intended for the stay of people and live-stock, and also in an industrial wastes used in the building  ;

- the control of the content of these isotopes, in this in particular:

a) the conditions that have to be met by laboratories leading the control regarding the system on the assurance of the quality of performed investigations, equipment, housing and environmental conditions and the frequency to participate in inter-laboratory comparative measurements,

b) the way of taking samples and their measurement, and factors taken into account when interpreting the results of the measurement.

§ 2.1 The content of natural radioisotopes: potassium K-40, radium Ra-226 and thorium Th-228 in raw materials and materials to be used in buildings intended for the stay of people and livestock, and also in industrial wastes to be used in the construction industry are to be set up using:

1) f1 activity coefficient defining the content of natural radioisotopes, and

2) f2 activity coefficient defining the content of radium Ra-226.

2. The activity coefficients referred to in point 1, are defined by the formulas:

Sk		               SRa		    STh

1) f1 = ----------------  +  ------------------ + --------------- ,

3 000 Bq/kg		300 Bq/kg	200 Bq/kg

2) f2 = SRa ,

where:

Sk, SRa and STh mark the respective radioactive concentration of the isotopes of potassium K-40, radium Ra-226 and thorium Th-228, expressed in becquerels on the kilogramme (Bq/kg).

§ 3. The values of the coefficients of the activity f1 and f2 may not exceed the values by more than 20%:

1) f1 = 1 and f2 = 200 Bq/kg in relation to raw materials and building materials applied in buildings designed for stay of people and livestock;

2) f1 = 2 and f2 = 400 Bq/kg in relation to industrial wastes applied in ground-based buildings erected in developed areas or in areas designed to be built-up pursuant to a local build-up plan and to levelling of such areas ;

3) f1 = 3.5 and f2 = 1.000 Bq/kg in relation to industrial wastes applied in ground-based parts of structures not mentioned in the section 2 and to level areas unmentioned in point 2;

4) f1 = 7 and f2 = 2.000 Bq/kg in relation to industrial wastes applied in underground parts of buildings referred to in point 3, and in underground structures, including railway and road tunnels, excluding industrial wastes made use in underground mining excavations.

§ 4. When industrial wastes are applied to levelling the areas referred to in clause 3 point 2 and 3, and to construction of roads, sports and recreation facilities, the absorbed dose shall be limited to 0,3 µGy/h at the altitude of 1 m above ground, road or site level, in particular by applying an additional layer of another material, while maintaining the required value of f1 and f2 coefficients.

§ 5. The content of natural radioisotopes potassium K-40, radium Ra-226 and thorium Th-228 in raw materials and components applied in buildings for the stay of people and livestock, as well as in industrial wastes applied in construction, shall be controlled by laboratories accredited to perform such control and laboratories which:

1) have implemented a quality assurance system in respect of the performed tests including:

a) the general description of the applied testing methodology,

b) the documentation of measuring devices and technical equipment,

c) the instructions for taking and preparation of samples,

d) the instructions for spectrometer operation,

e) the measurement instructions, including measurements to verify the correct operation of measuring devices,

f) the way of recording the measurement results,

g) the examples reports from supervisory investigations and the records of results for these investigations,

h) requirements relating to the personnel qualifications;

2) they are equipped with:

a) the devices to preparing (crumbling, drying, sifting and weighing) of samples,

b) the gamma radiation spectrometer with a scintillating or semiconductor detector enabling to measure the content of natural radioisotopes in samples,

c) the protective house enabling to locate a detector in it and a vessel with a sample,

d) the sample sources to energetic calibration and the control of the stability of the spectrometer work,

e) in the reference patterns having natural radioisotopes serving the calibration of the spectrometer;

3) they possess place and environmental conditions necessary to make use, according to operational instructions, the measuring devices and technical equipment serving to perform the tests;

4) they participate, on the own cost, in interlaboratory comparative measurements organised by the President of the National Atomic Energy Agency, at least on three years.

§ 6.1. The sampling procedure for the purpose of control, referred to in clause 5, shall be conducted at a frequency dependent on the technological process and to ensure that representative samples are taken.

2. The controlled raw material, component or waste material shall be dried to dry mass and crumbled to produce grains of less than 2 mm prior to the location in the measurement vessel.

3. If radium Ra-226 is labelled in the sample by measuring the radiation of its disintegration products, prior to the measurement, the measurement vessel with the sample shall be tightly closed for a period of at least:

1) 5 days, if the controlled raw material, component or waste is of natural origin and has not been processed or stored for minimum 30 days, or

2) 14 day, if the controlled raw material, component or waste has been processed.

4. The mineral composition and the content of natural radioisotopes in reference to samples applied for the calibration of measurements corresponds to the controlled raw materials, components or waste.

5. Measurement results shall be interpreted in terms of:

1) radiation background not generated by the measured sample;

2) impact of the radiation of isotopes other than those measured in the sample;

3) the effect of self-absorption of radiation in the sample.

6. The total uncertainty of the determination of f1 and f2 activity indicators at confidence level of 0.95 shall not exceed 20% of their value if not less than 0.8 of the value of the indicators specified in §3.

§ 7. The regulation of the Council of Ministers of 3 December 2002 on the requirements imposed on the content of natural radioisotopes in raw materials and components applied in buildings for the stay of people and livestock, and in industrial waste applied in construction, and the control of radioisotope content (Journal of Laws No. 220, item 1850) becomes invalid.

§ 8. This regulation becomes effective after 14 days since its announcement.

President of the Council of Ministers: J. Kaczyński

Changes of the uniform text of the above Act were published in the Journal of Laws of 2004, No. 173, item 1808, of 2005, No. 163, item 1362 and of 2006, No. 52, item 378, No. 104, item 708, No. 133, item 935 and No. 170, item 1217.