CONSTRUCT 09/853

The Classification Without Further Testing (CWFT) possibilities against essential characteristics other than fire and dangerous substances

1.	Introduction

The mechanism allowing the classification without further testing has been successfully used up until now in the frame of the harmonisation activity related to Council Directive 89/106/EEC (CPD).

Although not specifically established in the CPD, the CWFT mechanism has been, up until now, used exclusively for assigning a specific reaction to fire class to construction products showing a stable performance in relation to their fire behaviour.

The reasons for this “de facto” limitation to the reaction to fire aspects is mainly due to the relative “easiness” of dealing with an issue (the fire behaviour) which was already quite well consolidated from the regulatory point of view at national level even before the entering into force of the CPD and, afterwards, by the well established European classification valid in, and applied by all the EU members.

However, even if the CPD itself does not contain specific provisions related to that mechanism, put in place to avoid useless repetitions of expensive tests, it does not prevent the possibility of using it, whenever possible, also for other essential characteristics related to the essential requirements of the works, where the repetitions of tests is not justified.

This possibility is also recognised by the proposed Regulation laying down harmonised conditions for the marketing of the construction products - CPR (COM(2008) 311 final).

The CPR has the scope to ensure that reliable information is presented in relation to the performances of construction products related to the basic work requirements, when placed on the EEA market. This is expected to be achieved by providing a common technical language to be used by manufacturers when placing products on the market and by public authorities when defining the technical requirements of works which influence, either directly or indirectly, the products to be used in those works.

This common technical language is set out in the harmonised technical specifications (harmonised European standards - hEN and European Assessment Documents - EAD developed in the frame of the proposed Regulation.

Amongst the measures included in the proposed text, aimed at simplifying the route to CE marking reducing the administrative burden of manufacturers (including micro and small enterprises), an encouragement is addressed to specification writers (CEN/CENELEC and Technical Assessment Bodies) in order:

● 	to replace testing in the harmonised technical specifications by other less onerous methods, such as descriptive methods, and

●  	to introduce classes, as far as possible, in the harmonised standards in order to avoid unnecessary testing of construction products, for which performance has already sufficiently been demonstrated by stable previous test results or other existing data. Therefore, manufacturers could be allowed, under the conditions set up in the harmonised technical specifications or in a Commission Decision, to declare a certain level or class of performance without further testing.

This document intends to propose a procedure, similar to the one already in place regarding the fire behaviour of construction products (reaction to fire - external fire performance for roofs), making possible, whether accepted, future applications for classifying without further testing a given performance of construction products related to essential characteristics other than fire and dangerous substances, for which the relevant harmonised product standards set up classes of performance.

The intention is to start since now the dissemination amongst specification writers of the opportunity to start considering the inclusion of technical classes in the harmonised product standards under revision, so that to have them progressively aligned with this principle of the new Regulation.

Even if still under discussion in the European Parliament and the Council, it is highly probable that possible changes due to the discussion in progress will not disregard this simplification measure.

2.	The concept of the classification without further testing

The classification without further testing (CWFT) is possible whether a construction product has proven to have a stable behaviour of a given characteristic in a given class of performance set up by the applicable harmonised product standard.

The evidence of such a stable behaviour shall be demonstrated:

- 	in accordance with the appropriate European test method;

- 	within the variability in the manufacture process allowed by the harmonised product standard;

- 	when evaluated for the influence of other possible variations that may occur outside the scope of the specification, which may have an impact on the performance of the given essential characteristic for which the classification is claimed.

Examples of technical classes already included in harmonised product standards are given in Annex 3.

Only generic construction products and not proprietary products, are suitable for the possible classification without further testing. The classification is assigned by a Commission Decision in accordance with the established procedure, following the positive technical achievement of the application.

The Commission considers any application for the possible CWFT duly submitted by an applicant. In doing so, the Commission may ask the view of expert groups and/or sub-groups on how to progress the request from a technical point of view.

3.	The conditions for the classification without further testing other than fire

The classification without further testing (CWFT) of a performance of construction products other than fire shall only be possible provided that the following conditions are fulfilled:

a) - 	the harmonised product standard covering the product subject to a CWFT application is adopted at the CEN /CENELEC level and its reference is published in OJEU;

b) - 	the harmonised product standard establishes classes to be used for expressing a given performance;

c) - 	the test method to be carry out for the given essential characteristic is directly or by reference included in the harmonised product standard;

d) - 	the mounting and fixing conditions of samples for the carry out of the test on the given essential characteristic are fully defined, taking into account the specific end use condition of the construction product, in the harmonised product standard or in the adopted European supporting standard or in a separate adopted European standard;

e) -	the tests are carried out by a body duly notified for these tests.

The possible classification without further testing claimed is assigned after the necessary technical checks and it is established by a Commission Decision published in OJEU.

# 4.	Construction products suitable to be classified without further testing

The classification without further testing may refer to any construction product of known and stable performance for defined end use applications with respect to any essential characteristic included in Table ZA.1 of Annex A of the applicable harmonised product standard in which technical classes are indicated for expressing test results and provided that the conditions from a) to e) referred to in clause 2 are met.

Construction products have to be intended as product families, product sub-families and generic products as defined in Guidance Paper G and specified by European product standards. It may also be possible to extend the CWFT concept to kits, if it is possible to define them with sufficient precision.

Construction products for which a clearly identifiable stage in the production process could results in an improvement of the performance of the essential characteristic concerned by the CWFT request cannot be considered eligible for the CWFT.

Construction products for which a CWFT application is introduced, shall:

- demonstrate by test to be stable in the claimed class;

- demonstrate to satisfy the claimed class at the end of tests;

- be defined with sufficient precise description.

# 5.	The procedure for the CWFT request

A CWFT request, or a request to modify/amend an existing established classification, has to be submitted to the Construction Unit of the European Commission by any interested party, i.e. Member State, CEN/CENELEC Technical Committees or European industry federations.

However, requests should be introduced, wherever possible, by CEN/CENELEC. Applications from individual manufacturers are not allowed, even though the results might apply to only one or a few manufacturers.

To avoid unnecessary work and expense for applicants, the procedure comprises the following two steps:

- 1	Application letter addressed to the Commission to get its advice on whether the construction product in question is in principle suitable to be classified without further testing.

- The letter should contain:

- -	a short description of the construction product;

- -	the reference to the harmonised product standard;

- -	the indication of the essential characteristic for which the classification is claimed without however indicating the claimed class(es).

- In order to express its advice, the Commission may request the view of its expert group.

- Once the case has been positively considered in principle by the Commission, the applicant will be requested to prepare the technical file supporting the application and containing all the information indicated in the specific template (see Annex 1), addressing it to the Commission.

- 2	Detailed analysis of the technical file.

- The Commission may set up expert groups requesting their view on whether the information contained in the technical file are sufficient or whether further technical information need to be requested to the applicant.

# 6.	Preparation of the request

The level of proof needed to accompany a request will vary case by case, depending upon e.g. how much historical experience/data is available, or whether products are already recognised in national systems.

Any request must be accompanied by some test results from the appropriate European test method standard(s), the tests having been performed under the same conditions (e.g. with regard to the number of samples tested) as would be required for normal classification by testing.

However, existing national test method results (especially where these are close to the European test), may be used as supplementary evidence, for example as a starting point to establish the “range” of product performance.

The intention is not to impose an unnecessary burden upon industry, whilst ensuring that sufficient information is supplied upon which the expert group can form an opinion.

It is recommended that, once a case has been positively considered in principle by the Commission, the applicant shall establish exactly which products, and in which end-use applications, are to be covered.

Initial responsibility for specifying the range of their products belongs to the applicant. The CWFT sub-group may need to identify variability in the product in relation to parameters not defined in the technical specification and, as a consequence, limitations on the variability of these additional parameters may need to be added to the specification.

The scope of CWFT may be more limited than the scope of the technical specification itself.

With respect to some test methods, any case has to consider the end use conditions of the product, bearing in mind that test results obtained from less favourable end use conditions will be applicable to more favourable conditions, and therefore that the scope of the CWFT case will be larger.

Products claiming to be classified without further testing must be clearly above the lower class limits, to provide a safety margin. This should be determined on a statistical basis in relation to the scattering of results.

In general terms, each classification parameter should be either 20% above the class limit (although some relaxation of this may be possible), or shown by statistical means to have a satisfactory safety level, for a request to be accepted.

Due account will be taken of the likely variability in the production process of products.

- It is the responsibility of the applicant to agree to allow the notified laboratory to make all information available to CWFT sub-group members.

- The final report (in electronic format) containing all the detailed information has to be prepared by the applicant following the template (see Annex 2). Whether the required details are not supplied in appropriate way, the application will not be handled any further.

7.	Decisional procedure

Once the CWFT expert group/sub-group set up by the Commission has expressed the view that the case:

- 	is completed;

- 	the product is sufficiently well described in relation to the variability of its parameters and the effect this variability may have on its fire performance;

- 	sufficient test results have been provided to justify the case,

the final report is sent to the Commission for its further handling in the framework of the CPD procedures (draft decision to be submitted to the vote of the Member States represented in the SCC – adoption by the Commission – publication in OJEU).

Whether the expert group/sub-group will express the opinion that further work needs to be done in support of the case, the Commission will inform the applicant accordingly.

8.	Disputes

The applicant is responsible for supplying the Commission with all the information required to make a balanced judgement.

Following the initial submission, the expert group/sub-group may need additional information, and in such cases the applicant will be asked, by the Commission, to provide this.

If there is reasonable doubt that the proposition is not valid, either during the first stage of agreement in principle or during the second stage of detailed examination, then the expert group/sub-group will advice the Commission on the opportunity to reject the request.

In such an event, the discussion on the case would not normally be re-opened. This should encourage applicants to support their requests in appropriate ways.

However, the applicant may appeal, if they can provide the Commission with additional supporting information.

9.	Responsibilities

The procedure described above is intended to ensure, with a high degree of certainty, that any product object of a CWFT decision has the claimed performance class associated with it.

However, if experience shows that a mistake has been made in the Decision, this should immediately be brought to the attention of the European Commission. This may result in either an amendment or deletion of the published Decision.

The manufacturer is always responsible for the CE marking of his product (Article 4.6 of Council Directive 89/106/EEC), and therefore regardless of the fact that the product is covered by a CWFT decision, the manufacturer takes the responsibility that any performance class declared in the CE marking is correctly covered by the corresponding physical characteristics of the product.

The product must correspond to the product description given in the relevant CWFT decision within the limits defined and the performance of the product should correspond to the given class.

The normal safeguard procedures of the Council Directive 89/106/EEC apply if a manufacturer is considered to have incorrectly applied the CE marking (e.g. by claiming his product meets the criteria of a CWFT class when it does not). Member States also have market surveillance obligations in this respect.

Once a product appears on a published CWFT decision, there is no third party intervention to “verify” that the product actually corresponds to the description given in it. However, for products for which a clearly identifiable stage in the production process results in an improvement of the claimed performance, a notified certification body has the responsibility to certify that the product corresponds to the product specification given in the Commission Decision (i.e. the product is considered to be under attestation of conformity level 1).

A notified body should inform manufacturers of the applicability of CWFT decisions in relation to their products, as manufacturers may not be aware.

However, CWFT decisions represent the minimum performance class as applied to a product, and therefore, a manufacturer is free to demonstrate a higher performance class for his product on the basis of test results and a performance classification report.

Annex 1

TEMPLATE FOR PREPARING THE TECHNICAL FILE

1 	The product

1.1	Detailed description of the product.

1.1	Detailed description of the composition of the product (including materials, elements, components, ancillaries).

1.2	Permissible or likely variations in the product during the manufacture.

1.3	Intended use of the product.

1.4	End use(s) conditions of the product.

2	The Classification

2.1	Essential characteristic for which the product performance is considered to be stable in relation to the envisaged end use(s).

2.2	The performance class(es) claimed.

2.3	Reference to the harmonised product standard(s).

2.4	Reference to the European supporting standard(s) and European standard(s) (where relevant).

2.5	Parameters affecting the performance of the essential characteristic claimed.

3	Historical data

3.1	Copy of test reports containing the results of European test methods carried out before the CWFT application is introduced and regarding the same essential characteristic for which the performance classification is claimed.

- 3.2	Copy of test reports containing the results of national test methods (if relevant).

3.3	Proof of presence of the product on national lists, where this exists.

4	The test programme

4.1	List of the test proposed for testing the performance related to the essential characteristic (including the number of tests envisaged, the number of samples to be tested and the reference to the European standard describing the test).

4.2	Procedure and responsibilities for selecting test samples.

Annex 2

TEMPLATE FOR THE PREPARATION OF THE FINAL CWFT REPORT

(Cover page)

Request of

……………………………………..…………………………………………...….……….

(name of the applicant)

to classify without further testing

…………………………………………………………………………………………….

(generic name of the product covered by the application)

for use in accordance with

…………..………………………………………………………..…………………………

(reference of the harmonised product standard/s)

CLASSIFIED WITHOUT FURTHER TESTING REPORT

.……………………..….

(date)

1 	The product

1.1	Detailed description of the product.

1.1	Detailed description of the composition of the product (including materials, elements, components, ancillaries).

1.2	Permissible or likely variations in the product during the manufacture.

1.3	Intended use of the product.

1.4	End use(s) conditions of the product.

2	The Classification

2.1	Essential characteristic in which the product performance is considered to be stable in relation to the envisaged end use(s).

2.2	The performance class(es) claimed.

2.3	Reference to the harmonised product standard(s).

2.4	Reference to the European supporting standard(s) and European standard(s) (where relevant).

2.5	Parameters affecting the performance of the essential characteristic claimed.

3	Historical data

3.1	Copy of test reports containing the results of European test methods carried out before the CWFT application is introduced and regarding the same essential characteristic for which the performance classification is claimed.

- 3.2	Copy of test reports containing the results of national test methods (if relevant).

3.3	Proof of presence of the product on national lists, where this exists.

4	The agreed test programme

4.1	List of the tests proposed for the testing the performance related to the essential characteristic (including the number of tests envisaged, the number of samples to be tested and the reference to the European standard describing the test).

4.2	Procedure and responsibilities for selecting test samples.

4.3	Mounting and fixing conditions under which the proposed test will be carried out.

5	Testing requirements for the classification

6	Test evidence submitted

6.1	…………….

6.2	…………….

7	Potential influence of variation in product parameters on the performance test

8	CONCLUSIONS

8.1	Statement of product and class sought

8.2	Proposed table for inclusion in Commission Decision

Table – Technical classes of …………..…….. performance for …………..…….

| Product | Product detail | Technical class (1) |
| --- | --- | --- |

(1) Technical class as provided in clause ………. of the harmonised product standard

Annex 3

EXAMPLES OF TECHNICAL CLASSES INCLUDED IN HARMONISED PRODUCT STANDARD

The following non exhaustive list provides an overview of some harmonised product standards (OJEU C321 of 16.12.2008) where technical classes are established.

●	EN 12794:2005 “Precast concrete products - Foundation piles”

- 	Strength classes of hardened concrete

-	Compressive strengths of hardened concrete

●	EN 494:2004+A3:2007 “Fibre-cement profiled sheets and fittings - Product specification and test methods”

-	Breaking load

-	Bending moment

●	EN 14688:2006 “Sanitary appliances - Wash basins – Functional requirements and test methods”

- 	Flow rate of overflow

●	EN 14509:2006 “Self-supporting double skin metal faced insulating panels - Factory made products – Specifications”

-	Water permeability

●	EN 14449:2005 “Glass in building - Laminated glass and laminated safety glass - Evaluation of conformity/Product standard”

-	Bullet resistance

-	Explosion resistance

-	Burglar resistance

-	Pendulum body impact resistance

●	EN 14351-1:2006 “Windows and doors - Product standard, performance characteristics - Part 1: Windows and external pedestrian doorsets without resistance to fire and/or smoke leakage characteristics”

-	Watertightness

-	Resistance to wind load

-	Impact resistance

-	Ability to release

-	Operating forces for automatic devices

-	Air permeability

●	EN 14080:2005 “Timber structures - Glued laminated timber – Requirements”

-	Modulus of elasticity

-	Bending strength

-	Compressive strength

-	Tensile strength

-	Shear strength

●	EN 13830:2003 “Curtain walling - Product standard”

-	Watertightness

-	Impact resistance

-	Air permeability

● 	EN 13748-2:2004 “Terrazzo tiles - Part 2: Terrazzo tiles for external use

-	Weathering resistance”

-	Breaking strength

●	EN 13659:2004 “Shutters - Performance requirements including safety”

-	Resistance to wind loads

The Commission service is examining the possible extension of the CWFT concept to dangerous substances. These are therefore excluded by this proposal.

A class of performance is a technical class as defined in Guidance Paper E (revision September 2002).