| Annex I for an amendment of Mandate M/103 Thermal Insulation Products with ER3 related requirements, August 2009 NOTE: this is a general list of notified regulations which may relate to regulated dangerous substances and ER 3 but which also deals with other issues. TC 88 is asked to check through these and establish which regulations and which substances or properties are relevant to this specific task and to cover them in its standards. Further, in the below list of regulations, the different thermal insulation types are not always aligned with the definitions of types in the thermal insulation product standards so TC 88 is asked to adjust as appropriate. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NOTE: In regard to the Essential Requirement 3 the requirements deriving from national regulations and specified in the "Indicative List" have to be fulfilled. If a substance mentioned below is not used in a product it can be closed out in the product description. In this case a test is not necessary. If further substances from the Indicative List not listed in this annex are used or present in the product (in the thermal insulation product itself or in its cladding/lamination), they must be added to this mandate. Substances included in the candidate list of substances of very high concern for authorisation under REACH are marked bold (see: http://echa.europa.eu/chem_data/candidate_list_table_en.asp). Current intentions for proposals for substances of very high concern can be consulted under: http://echa.europa.eu/chem_data/reg_int_tables/reg_int_curr_int_en.asp |  |  |  |  |  |
| Thermal insulation products for buildings |  |  |  |  |  |
| Standards published in the Official Journal of the European Union |  |  |  |  |  |
| EN Number | Standard Title | Requirements for roofs, exposed to weathering, and for foundations and retaining walls, in contact with soil | Requirements for ceilings, floors, internal walls and partitions (contact with indoor air possible) | Requirements for roofs, protected from weathering, for external walls and claddings (no contact with indoor air) | Regulations |
| EN 13162 | Thermal insulation products for buildings — Factory made mineral wool (MW) products — Specification | not applicable | VOC, formaldehyde, fibre composition | fibre composition | Directive 97/69/EC, 1998-156-D, 2007-90-DK (fibre composition); 2008-273-FIN, 2005-255-D, PL-1996 (formaldehyde, certain VOC) |
| EN 13163 | Thermal insulation products for buildings — Factory made products of expanded polystyrene (EPS) — Specification | not applicable | VOC, styrene, pentane, hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | Regulation 1907/2009/EC (candidate list: HBCD, SCCP); 2008-273-FIN (styrene, VOC, pentane), PL-1996 (styrene, certain VOC); 2007-9016-N (HBCD, MCCP); Decision 2007/395/EC (SCCP) |
| EN 13164 | Thermal insulation products for buildings — Factory made products of extruded polystyrene foam (XPS) — Specification | hydrofluorocarbons (HFC), hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (SCCP, MCCP) | hydrofluorocarbons (HFC), styrene, pentane, VOC, hexabromocyclododecane (HBCD), tributylphosphate (TBP), triphenylphosphate (TPP), medium and short chained chlorinated paraffins (SCCP, MCCP) | hydrofluorocarbons (HFC), hexabromocyclododecane(HBCD), medium and short chained chlorinated paraffins (SCCP, MCCP) | Regulation 1907/2009/EC (candidate list: HBCD, SCCP); 2001-121-DK, 2005-485-A (HFC); 2007-9016-N (HBCD, MCCP), Decision 2007/395/EC (SCCP); 2008-273-FIN (styrene, VOC, pentane, TBP, TPP), PL-1996 (styrene, certain VOC) |

| EN Number | Standard Title | Requirements: in contact with soil | Requirements: contact with indoor air possible | Requirements: no contact with indoor air or soil | Regulations |
| --- | --- | --- | --- | --- | --- |
| EN 13165 | Thermal insulation products for buildings — Factory made rigid polyurethane foam (PUR) products — Specification | not applicable | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, pentane, tributylphosphate, triphenylphosphate, trichloroethylene, benzene, cadmium | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, DEHP, DBP, BBP, Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, cadmium | Regulation 552/2009/EC (pentabromodiphenylether, octabromodiphenylether, cadmium); Regulation 1907/2009/EC (candidate list: DEHP, DBP, BBP); Commission Decision 2009 /425/EC (organostannic compounds); PL-1996 (DBP): 2005-9020-N (decabromo-diphenylether); 2001-121-DK, 2005-485-A (HFC); 2008-273-FIN (pentane, isocyanates, TBP, TPP), 2007-9016-N (cadmium), 2009-34-F (trichloroethylene); 2009-104-F (benzene, DEHP, DBP) |
| EN 13166 | Thermal insulation products for buildings — Factory made products of phenolic foam (PF) Specification | not applicable | phenol, formaldehyde | none: to be confirmed through a WT application | 2008-273-FIN, PL-1996, 2005-255-D (phenol, formaldehyde, VOC according to the target list) |
| EN 13167 | Thermal insulation products for buildings — Factory made cellular glass (CG) products — Specification | none | none | none |  |
| EN 13168 | Thermal insulation products for buildings — Factory made wood wool (WW) products Specification | not applicable | VOC, formaldehyde, pentachlorophenol (recycled wood), declaration of biocides | Pentachlorophenol (recycled wood), declaration of biocides | 2008-273-FIN, PL-1996 (VOC, formaldehyde, PCP), Decisions 94/783/EC, 1996/211/EC, 1999/831/EC, 2007-9016-N (PCP), Directive 98/8/EC (biocides) |
| EN 13169 | Thermal insulation products for buildings — Factory made products of expanded perlite (EPB) — Specification | not applicable | radioactivity, PAH (e.g. benzo(a)pyrene, naphthalene), VOC | none: to be confirmed through a WT application | 2008-273-FIN (VOC, naphthalene, benzo(a)pyrene), PL-1996 (VOC, naphthalene); Directive 96/29/Euratom (radiation) |
| EN Number | Standard Title | Requirements: in contact with soil | Requirements: contact with indoor air possible | Requirements: no contact with indoor air or soil | Regulations |
| EN 13170 | Thermal insulation products for buildings — Factory made products of expanded cork (ICB) — Specification | not applicable | VOC, declaration of biocides, PAH (e.g. naphthalene, benzo(a)pyrene) | declaration of biocides | 2008-273-FIN (VOC, naphthalene, benzo(a)pyrene), PL-1996 (VOC, naphthalene); Directive 98/8/EC (biocides) |
| EN 13171 | Thermal insulating products for buildings — Factory made wood fibre (WF) products — Specification | not applicable | VOC, formaldehyde, declaration of biocides, PAH (e.g. naphthalene, benzo(a)pyrene), pentachlorophenol (PCP) | declaration of biocides, PCP (recycled wood) | 2008-273-FIN (VOC, formaldehyde, PCP, naphthalene, benzo(a)pyrene), PL-1996 (VOC, formaldehyde, PCP, naphthalene), Decisions 94/783/EC, 1996/211/EC, 1999/831/EC, 2007-9016-N (PCP), Directive 98/8/EC (biocides) |
| EN 14063–1 | Thermal insulation products for buildings — Insitu formed expanded clay lightweight aggregate products — Part 1: Specification for the loosefill products before installation | not applicable | none: to be confirmed through a WT application | none: to be confirmed through a WT application | relevant regulated dangerous substances, if any |
| EN 14316–1 | Thermal insulation products for buildings — Insitu thermal insulation formed from expanded perlite (EP) products — Part 1: Specification for bonded and loosefill products before installation | not applicable | radioactivity, PAH (e.g. naphthalene), VOC | none: to be confirmed through a WT application | 2008-273-FIN (VOC, naphthalene, benzo(a)pyrene), PL-1996 (VOC, naphthalene); Directive 96/29/Euratom (radiation) |
| EN 14317–1 | Thermal insulation products for buildings — Insitu thermal insulation formed from exfoliated vermiculite (EV) products — Part 1: Specification for bonded and loosefill products before installation | not applicable | radioactivity | none: to be confirmed through a WT application | Directive 96/29/Euratom (radiation) |
| EN Number | Standard Title | Requirements: in contact with soil | Requirements: contact with indoor air possible | Requirements: no contact with indoor air or soil | Regulations |
| Failed Formal Vote or Unique Acceptance Procedure Drafts (CEN snapshot June 2009) |  |  |  |  |  |
| EN 14315–1 | Thermal insulating products for buildings — Insitu formed sprayed rigid polyurethane (PUR) and polyisocyanurate (PIR) foam products — Part 1: Specification for the rigid foam spray system before installation | not applicable(to be confirmed by CEN/TC 88) | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, pentane, tributylphosphate, triphenylphosphate, cadmium | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, cadmium | Regulation 552/2009/EC (pentabromodiphenylether, octabromodiphenylether, cadmium); Regulation 1907/2009/EC (candidate list: DEHP, DBP, BBP); Commission Decision 2009/425/EC (organostannic compounds); 2001-121-DK, 2005-485-A (HFC); 2008-273-FIN (pentane, isocyanates, TBP, TPP); 2005-9020-N, 2009-167-D (decabromodiphenylether); 2007-9016-N (cadmium); 2009-104-F (DEHP, DBP, benzene); PL-1996 (DBP); 2009-34-F (trichloroethylene) |
| EN 14318–1 | Thermal insulating products for buildings — Insitu formed dispensed rigid polyurethane (PUR) and polyisocyanurate (PIR) foam products — Part 1: Specification for the rigid foam dispensed system before installation | not applicable(to be confirmed by CEN/TC 88) | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, pentane, tributylphosphate, triphenylphosphate, cadmium | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, cadmium | Regulation 552/2009/EC (pentabromodiphenylether, octabromodiphenylether, cadmium); Regulation 1907/2009/EC (candidate list: DEHP, DBP, BBP); Commission Decision 2009/425/EC ( organostannic compounds); 2001-121-DK, 2005-485-A (HFC); 2008-273-FIN (pentane, isocyanates, TBP, TPP); 2005-9020-N, 2009-167-D (decabromodiphenylether); 2007-9016-N (cadmium); 2009-104-F (DEHP, DBP, benzene); PL-1996 (DBP); 2009-34-F (trichloroethylene) |
| EN Number | Standard Title | Requirements: in contact with soil | Requirements: contact with indoor air possible | Requirements: no soil or indoor air contact | Regulations |
| Drafts passed CEN Enquiry (CEN snapshot June 2009) |  |  |  |  |  |
| EN 14064–1 | Thermal insulation products for buildings — Insitu formed loosefill mineral wool (MW) products — Part 1: Specification for the loosefill products before installation | not applicable | VOC, formaldehyde, fibre composition | fibre composition | Directive 97/69/EC, 1998-156-D, 2007-90-DK (fibre composition); 2008-273-FIN, PL-1996 (formaldehyde, VOC) |
| EN 15101–1 | Thermal insulation products for buildings — Insitu formed loosefill cellulose products — Part 1: Specification for the products before installation | not applicable | declaration of biocides | declaration of biocides | Directive 98/8/EC (biocides) |
| Thermal insulating products for building equipment and industrial installations |  |  |  |  |  |
| Failed Formal Vote or Unique Acceptance Procedure Drafts (CEN snapshot June 2009) |  |  |  |  |  |
| EN Number | Standard Title | Requirements for piping and other applications exposed to weathering or in contact with soil | Requirements for space heating, cooling and air conditioning and other applications in possible contact with indoor air | Requirements for applications with no contact to indoor air / soil and not exposed to weathering | Regulations |
| EN 14319–1 | Thermal insulating products for building equipment and industrial installations — Insitu formed dispensed rigid polyurethane (PUR) and polyisocyanurate (PIR) foam products — Part 1: Specification for the rigid foam dispensed system before installation | not applicable(to be confirmed by CEN/TC 88) | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, pentane, tributylphosphate, triphenylphosphate, cadmium | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, cadmium | Regulation 552/2009/EC (pentabromodiphenylether, octabromodiphenylether, cadmium); Regulation 1907/ 2009/EC (candidate list: DEHP, DBP, BBP); Commission Decision 2009/ 425/EC (organostannic compounds); 2001-121-DK, 2005-485-A (HFC); 2008-273-FIN (pentane, isocyanates, TBP, TPP); 2005-9020-N, 2009-167-D (decabromo-diphenylether); 2007-9016-N (cadmium); 2009-104-F (DEHP, DBP, benzene); PL-1996 (DBP); 2009-34-F (trichloroethylene) |
| EN Number | Standard Title | Requirements: in contact with soil / weathering | Requirements: contact with indoor air possible | Requirements: no contact with indoor air or soil | Regulations |
| EN 14320–1 | Thermal insulating products for building equipment and industrial installations — Insitu formed sprayed rigid polyurethane (PUR) and polyisocyanurate (PIR) foam products — Part 1: Specification for the rigid foam spray system before installation | not applicable(to be confirmed by CEN/TC 88) | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, pentane, tributylphosphate, triphenylphosphate, cadmium | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, cadmium | Regulation 552/2009/EC (pentabromodiphenylether, octabromodiphenylether, cadmium); Regulation 1907/2009/EC (candidate list: DEHP, DBP, BBP); Commission Decision 2009/425/EC (organostannic compounds); 2001-121-DK, 2005-485-A (HFC); 2008-273-FIN (pentane, isocyanates, TBP, TPP); 2005-9020-N, 2009-167-D (decabromodiphenylether); 2007-9016-N (cadmium); 2009-104-F (DEHP, DBP, benzene); PL-1996 (DBP); 2009-34-F (trichloroethylene) |
| Drafts undergoing Formal Vote or Unique Acceptance Procedure (CEN snapshot June 2009) |  |  |  |  |  |
| EN 14303 | Thermal insulation products for building equipment and industrial installations — Factory made mineral wool (MW) products — Specification | not applicable(to be confirmed by CEN/TC 88) | VOC, formaldehyde, fibre composition | fibre composition | Directive 97/69/EC, 1998-156-D, 2007-90-DK (fibre composition); 2008-273-FIN, PL-1996 (formaldehyde, VOC) |
| EN 14304 | Thermal insulation products for building equipment and industrial installations — Factory made flexible elastomeric foam (FEF) products — Specification | not applicable (to be confirmed by CEN/TC 88) | VOC, benzo(a)pyrene, medium chained chlorinated paraffins (MCCP), short chained chlorinated paraffins (SCCP), lead | MCCP, SCCP, lead | Regulation 1907/2009/EC (candidate list: SCCP); 2008-273-FIN (VOC, benzo(a)pyrene, naphthalene), PL-1996 (VOC, naphthalene); 2007-9016-N (MCCP, lead), Decision 2007/395/EC (SCCP) |
| EN 14305 | Thermal insulation products for building equipment and industrial installations — Factory made cellular glass (CG) products — Specification | None | none | none |  |
| EN Number | Standard Title | Requirements: in contact with soil / weathering | Requirements: contact with indoor air possible | Requirements: no contact with indoor air or soil | Regulations |
| EN 14306 | Thermal insulation products for building equipment and industrial installations — Factory made calcium silicate (CS) products — Specification | none: to be confirmed through a WT application | none: to be confirmed through a WT application | none: to be confirmed through a WT application | relevant regulated dangerous substances, if any |
| EN 14307 | Thermal insulation products for building equipment and industrial installations — Factory made extruded polystyrene foam (XPS) products — Specification | hydrofluorocarbons (HFC), hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (SCCP, MCCP) | hydrofluorocarbons (HFC), styrene, pentane, VOC, hexabromocyclododecane (HBCD), tributylphosphate (TBP), triphenylphosphate (TPP), medium and short chained chlorinated paraffins (SCCP, MCCP) | hydrofluorocarbons (HFC), hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (SCCP, MCCP) | Regulation 1907/2009/EC (candidate list: HBCD, SCCP); 2001-121-DK, 2005-485-A (HFC); 2007-9016-N (HBCD, MCCP), Decision 2007/395 /EC (SCCP); 2008-273-FIN (styrene, VOC, pentane, TBP, TPP), PL-1996 (styrene, VOC) |
| EN 14308 | Thermal insulation products for building equipment and industrial installations — Factory made rigid polyurethane foam (PUR) and polyisocyanurate foam (PIR) products — Specification | not applicable (to be confirmed by CEN/TC 88) | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, pentane, tributylphosphate, triphenylphosphate, cadmium | hydrofluorocarbons (HFC), pentabromodiphenylether, octabromodiphenylether, decabromodiphenylether, diethylhexylphthalate (DEHP), dibutylphthalate (DBP), benzylbutylphthalate (BBP), Tri-substituted organostannic compounds, Dibutyltin (DBT) compounds, isocyanates, cadmium | Regulation 552/2009/EC (pentabromodiphenylether, octabromodiphenylether, cadmium); Regulation 1907/2009/EC (candidate list: DEHP, DBP, BBP); Commission Decision 2009/ 425/EC (organostannic compounds); PL-1996 (DBP); 2001-121-DK, 2005-485-A (HFC); 2008-273-FIN (pentane, isocyanates, TBP, TPP); 2005-9020-N, 2009-167-D (decabromo-diphenylether); 2007-9016-N (cadmium); 2009-104-F (DEHP, DBP, benzene); 2009-34-F (trichloroethylene) |
| EN 14309 | Thermal insulation products for building equipment and industrial installations — Factory made products of expanded polystyrene (EPS) — Specification | not applicable (to be confirmed by CEN/TC 88) | VOC, styrene, pentane, hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | Regulation 1907/2009/EC (candidate list: HBCD, SCCP); 2008-273-FIN (styrene, VOC, pentane), PL-1996 (styrene, certain VOC); 2007-9016-N (HBCD, MCCP); Decision 2007/395/EC (SCCP) |

| EN Number | Standard Title | Requirements: in contact with soil /weathering | Requirements: contact with indoor air possible | Requirements: no contact with indoor air or soil | Regulations |
| --- | --- | --- | --- | --- | --- |
| EN 14313 | Thermal insulation products for building equipment and industrial installations — Factory made polyethylene foam (PEF) products — Specification | not applicable (to be confirmed by CEN/TC 88) | VOC, polybrominated diphenylethers (PBDE), MCCP | PBDE, MCCP | Regulation 552/2009/EC (pentabromodiphenylether, octabromodiphenylether); PL-1996, 2008-273-FIN (VOC according to target list), 2007-9016-N (MCCP), 2005-9020-N (decabromodiphenylether) |
| EN 14314 | Thermal insulation products for building equipment and industrial installations — Factory made phenolic foam (PF) products — Specification | not applicable (to be confirmed by CEN/TC 88) | phenol, formaldehyde | none: to be confirmed through a WT application | 2008-273-FIN, PL-1996 (phenol, formaldehyde) |
| Drafts waiting for UAP (foreseen for CEN snapshot September 2009) |  |  |  |  |  |
| EN 15599-1 | Thermal insulation products for building equipment and industrial installations - In-situ thermal insulation formed from expanded perlite (EP) products - Part 1: Specification for bonded and loose-fill products before installation | not applicable | radioactivity, PAH (e.g. naphthalene), VOC | none: to be confirmed through a WT application | 2008-273-FIN (VOC, naphthalene, benzo(a)pyrene), PL-1996 (VOC, naphthalene); Directive 96/29/Euratom (radiation) |
| EN 15600-1 | Thermal insulation products for building equipment and industrial installations - In-situ thermal insulation formed from exfoliated vermiculate (EV) products - Part 1: Specification for bonded and loose-fill products before installation | not applicable | radioactivity | none: to be confirmed through a WT application | Directive 96/29/Euratom (radiation) |

| Thermal insulation products for civil engineering applications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Approved standards cited in the Official Journal |  |  |  |  |  |
| EN Number | Standard Title | Requirements for roads and railways and embankment filling and other applications exposed to weathering or in contact with soil | Applications in contact with indoor air | Requirements for roads and railways and embankment filling and other applications not exposed to weathering nor in contact with soil | Regulations |
| EN 14933 | Light weight fill and insulation products for civil engineering applications — Factory made products of expanded polystyrene (EPS) — Specification | Hexabromocyclo-dodecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | not applicable | hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | Regulation 1907/2009/EC (candidate list: HBCD, SCCP); 2007-9016-N (HBCD, MCCP); Decision 2007/395/EC (SCCP) |
| EN 14934 | Thermal insulation and light weight fill products for civil engineering applications — Factory made products of extruded polystyrene foam (XPS) — Specification | hydrofluorocarbons (HFC), hexabromocyclo-dodecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | not applicable | hydrofluorocarbons (HFC), hexabromocyclododecane (HBCD), medium and short chained chlorinated paraffins (MCCP, SCCP) | Regulation 1907/2009/EC (candidate list: HBCD, SCCP); 2001-121-DK, 2005-485-A (HFC); 2007-9016-N (HBCD, MCCP), Decision 2007/395/EC (SCCP) |
| Drafts passed CEN Enquiry (CEN snapshot April 2009 – further progress foreseen under a new work item number) |  |  |  |  |  |
| EN 15732 | Light weight fill and thermal insulation products for civil engineering applications (CEA) -Expanded clay lightweight aggregate products (LWA) | none: to be confirmed through a WT application | not applicable | none: to be confirmed through a WT application | relevant regulated harmful and dangerous substances, if any |