| Annex I to the amendment to Mandate M/125 "Aggregates" with respect to ER3 related requirements NOTE: this is a general list of notified regulations which may relate to regulated dangerous substances and ER 3 but which also deals with other issues. TC 154 is asked to check through these and establish which regulations and which substances or properties are relevant to this specific task and to cover them in its standards. Further, in the below list of regulations, the different aggregate types are not always aligned with the definitions of types in the aggregate product standards so TC 154 is asked to adjust as appropriate. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Harmonised Standards under M/125 cited in the Official Journal of the European Union |  |  |  |  |
| EN Number | Standard Title | Regulated field(s) of application | Requirements | Notified regulations. |
| EN 13139 | Aggregates for mortar | bound use in mortar for structural engineering | for recycled aggregates from crushed concrete and bricks Release: pH / electrical conductivity / chloride (Cl) / sulphate (SO4) / arsenic (As) / lead (Pb) / cadmium (Cd) / chromium (Cr) / copper (Cu) / nickel (Ni) / mercury (Hg) / zinc (Zn) / phenol Content: hydrocarbons / PAH / EOX / PCB / ammonium-N / nitrite-N for other recycled aggregates and certain manufactured aggregates, (such as steel slag, municipal waste incineration slag, fly ash and bottom ash from co-combustion) additionally Release: turbidity / tendency to produce foam / AOX / antimony (Sb) / barium (Ba) / boron (B) / cobalt (Co) / chromium VI / molybdenum (Mo) / selenium (Se) / thallium (Tl) / tin (Sn) / vanadium (V) / cyanide, (CN-) / fluoride (F-) / PAH / naphthalene and methylnaphthalenes / highly volatile halogenated hydrocarbons / 1,2-dichloroethane / tri- and tetrachloroethene / chloroethene (vinyl chloride) / alkylated benzenes / benzene / ethylbenzene / toluene / xylenes / MTBE / nonylphenol / chlorinated phenols / hexachlorobenzene / chlorobenzenes / epichlorohydrin Content: TOC / arsenic / lead / barium / cadmium / chromium (total) / cobalt / molybdenum / nickel / mercury / selenium / thallium / tin / vanadium / zinc / chloride / cyanide / fluoride / sulphate / PAH / / PCDDs and PCDFs / highly volatile halogenated hydrocarbons / PCBs / hydrocarbons / benzene / ethylbenzene / toluene / xylenes / radioactivity | 2005-424-D, 2006-90-D, 2007-653-A |
| EN 13055-1 | Lightweight aggregates - Part 1: Lightweight aggregates for concrete, mortar and grout | bound use in concrete and mortar for civil and structural engineering (excluding public road construction) | for recycled aggregates from crushed concrete and bricks Release: pH / electrical conductivity / chloride (Cl) / sulphate (SO4) / arsenic (As) / lead (Pb) / cadmium (Cd) / chromium (Cr) / copper (Cu) / nickel (Ni) / mercury (Hg) / zinc (Zn) / phenol Content: hydrocarbons / PAH / EOX / PCB / ammonium-N / nitrite-N for other recycled aggregates and certain manufactured aggregates, (such as steel slag, municipal waste incineration slag, fly ash and bottom ash from co-combustion) additionally Release: turbidity / tendency to produce foam / AOX / antimony (Sb) / barium (Ba) / boron (B) / cobalt (Co) / chromium VI / molybdenum (Mo) / selenium (Se) / thallium (Tl) / tin (Sn) / vanadium (V) / cyanide, (CN-) / fluoride (F-) / PAH / naphthalene and methylnaphthalenes / highly volatile halogenated hydrocarbons / 1,2-dichloroethane / tri- and tetrachloroethene / chloroethene (vinyl chloride) / alkylated benzenes / benzene / ethylbenzene / toluene / xylenes / MTBE / nonylphenol / chlorinated phenols / hexachlorobenzene / chlorobenzenes / epichlorohydrin Content: TOC / arsenic / lead / barium / cadmium / chromium (total) / cobalt / molybdenum / nickel / mercury / selenium / thallium / tin / vanadium / zinc / chloride / cyanide / fluoride / sulphate / PAH / / PCDDs and PCDFs / highly volatile halogenated hydrocarbons / PCBs / hydrocarbons / benzene / ethylbenzene / toluene / xylenes / radioactivity | 2005-424-D, 2006-90-D, 2007-653-A |

| EN Number | Standard Title | Regulated field(s) of application | Requirements | Notified regulations. |
| --- | --- | --- | --- | --- |
| EN 12620 | Aggregates for concrete | bound use in road construction | recycled aggregates and manufactured aggregates Release: pH / electrical conductivity / ammonium-N / nitrite-N / chloride (Cl) / sulphate (SO4) / cyanide / fluoride (F) / AOX / DOC / PAH (EPA) / phenol / arsenic (As) / lead (Pb) / cadmium (Cd) / chromium (Cr) / chromate / copper (Cu) / nickel (Ni) / mercury (Hg) / vanadium (V) / zinc (Zn) / PAH / phenol / mineral oil hydrocarbons Content: EOX, TOC, lead, cadmium, chromium (total), copper, zinc, PAH, hydrocarbons | 2004-71-D, 1999-263-A, 2007-385-A |
|  |  | bound use in concrete for civil and structural engineering (excluding public road construction) | for recycled aggregates from crushed concrete and bricks Release: pH / electrical conductivity / chloride (Cl) / sulphate (SO4) / arsenic (As) / lead (Pb) / cadmium (Cd) / chromium (Cr) / copper (Cu) / nickel (Ni) / mercury (Hg) / zinc (Zn) / phenol Content: hydrocarbons / PAH / EOX / PCB / ammonium-N / nitrite-N for other recycled aggregates and certain manufactured aggregates, (such as steel slag, municipal waste incineration slag, fly ash and bottom ash from co-combustion) additionally Release: turbidity / tendency to produce foam / AOX / antimony (Sb) / barium (Ba) / boron (B) / cobalt (Co) / chromium VI / molybdenum (Mo) / selenium (Se) / thallium (Tl) / tin (Sn) / vanadium (V) / cyanide, (CN-) / fluoride (F-) / PAH / naphthalene and methylnaphthalenes / highly volatile halogenated hydrocarbons / 1,2-dichloroethane / tri- and tetrachloroethene / chloroethene (vinyl chloride) / alkylated benzenes / benzene / ethylbenzene / toluene / xylenes / MTBE / nonylphenol / chlorinated phenols / hexachlorobenzene / chlorobenzenes / epichlorohydrin Content: TOC / arsenic / lead / barium / cadmium / chromium (total) / cobalt / molybdenum / nickel / mercury / selenium / thallium / tin / vanadium / zinc / chloride / cyanide / fluoride / sulphate / PAH / / PCDDs and PCDFs / highly volatile halogenated hydrocarbons / PCBs / hydrocarbons / benzene / ethylbenzene / toluene / xylenes / radioactivity | 2005-424-D, 2006-90-D, 2007-653-A |
| EN 13043 | Aggregates for bituminous mixtures and surface treatments for roads, airfields and other trafficked areas | bound use in road construction | recycled aggregates and manufactured aggregates Release: pH / electrical conductivity / ammonium-N / nitrite-N / chloride (Cl) / sulphate (SO4) / cyanide / fluoride (F) / AOX / DOC / PAH (EPA) / phenol / antimony (Sb) / arsenic (as) / barium (Ba) / lead (Pb) / cadmium (Cd) / chromium (Cr) / chromate / copper (Cu) / molybdenum (Mo) / nickel (Ni) / mercury (Hg) / selenium (Se) / vanadium (V) / zinc (Zn) / mineral oil hydrocarbons / PCB Content: EOX / TOC / arsenic / lead / cadmium / chromium / copper, zinc / PAH / PCB / hydrocarbons | 2004-71-D, 2005-735-FIN, 2006-223-E, 2007-385-A |

| EN Number | Standard Title | Regulated field(s) of application | Requirements | Notified regulations. |
| --- | --- | --- | --- | --- |
| EN 13055-2 | Lightweight aggregates - Part 2: Lightweight aggregates for bituminous mixtures and surface treatments and for unbound and bound applications | unbound use of any/all aggregates in contact with soil/groundwater and surface water | inorganic parameters (for all aggregates): Release: antimony (Sb) / arsenic (As) / barium (Ba) / cadmium (Cd) / chromium (Cr) / cobalt (Co) / copper (Cu) / mercury (Hg) / lead (Pb) / molybdenum (Mo) / nickel (Ni) / selenium (Se) / tin (Sn) / vanadium (V) / zinc (Zn) / bromide (Br) / chloride (Cl) / fluoride (F) / sulphate (SO4) further substances (for recycled and relevant manufactured aggregates): Release: pH / electrical conductivity / chromate / ammonium-N / nitrite-N / cyanide / AOX / DOC / PAH / hydrocarbons / phenol Content: EOX / TOC / arsenic / lead / cadmium / chromium / copper / zinc / benzene / ethylbenzene / toluene / xylenes (sum, being the sum of m-xylene, p-xylene and o-xylene) / phenol / naphthalene / phenanthrene / anthracene / fluoranthene / chrysene / benzo(a)anthracene / benzo(a)pyrene / benzo(k)fluoranthene / indeno(1,2,3cd)pyrene / benzo(ghi)perylene / PAHs (sum of the afore mentioned PAH and EPA PAH) / PCBs (sum of regulated congeners) / mineral oil /asbestos (weighed, serpentine asbestos plus amphibole asbestos) | 2006-557-NL, 2004-71-D, 2005-735-FIN, 2007-385-A |
|  |  | bound use in public road construction | recycled aggregates and manufactured aggregates Release: pH / electrical conductivity / ammonium-N / nitrite-N / chloride (Cl) / sulphate (SO4) / cyanide / fluoride (F-) / AOX / DOC / PAH (EPA) / phenol index / antimony (Sb) / arsenic (As) / barium (Ba) / lead (Pb) / cadmium (Cd) / chromium (Cr) / chromate / copper (Cu) / molybdenum (Mo) / nickel (Ni) / mercury (Hg) / selenium (Se) / vanadium (V) / zinc (Zn) / mineral oil hydrocarbons / PCB Content: EOX / TOC / arsenic / lead / cadmium / chromium / copper, zinc / PAH / PCB / hydrocarbons | 2004-71-D, 2005-735-FIN, 2007-385-A |
|  |  | bound use in other applications | recycled aggregates and manufactured aggregates Release: pH / electric conductivity / AOX / turbidity / tendency to produce foam / arsenic (As) / antimony (Sb) / barium (Ba) / boron (B) / cadmium (Cd) / chromium (Cr) / chromate / cobalt (Co) / copper (Cu) / lead (Pb) / mercury (Hg) / molybdenum (Mo) nickel (Ni) / selenium (Se) / thallium (Tl) / tin (Sn) / vanadium (V) / zinc (Zn) / cyanide (CN-) / fluoride (F-) / chloride (Cl) / sulphate (SO4) / / phenol / highly volatile halogenated hydrocarbons / 1,2-dichloroethane / tri- and tetrachloroethene / chloroethene (vinyl chloride) / alkylated benzenes / benzene / ethylbenzene / toluene / xylenes / MTBE / nonylphenol / chlorinated phenols / hexachlorobenzene / chlorobenzenes / epichlorohydrin / PAH / naphthalene and methylnaphthalenes / PCBs / hydrocarbons Content: EOX / TOC / arsenic / lead / barium / cadmium / chromium total / cobalt / molybdenum / nickel / mercury / selenium / thallium / tin / vanadium / zinc / chloride / cyanide / fluoride / sulphate / PAH / / PCDDs and PCDFs / highly volatile halogenated hydrocarbons / PCBs / hydrocarbons / benzene / ethylbenzene / toluene / xylenes / radioactivity | 2006-90-D |

| EN Number | Standard Title | Regulated field(s) of application | Requirements | Notified regulations. |
| --- | --- | --- | --- | --- |
| EN 13242 | Aggregates for unbound and hydraulically bound materials for use in civil engineering work and road construction | unbound use of any/all aggregates in contact with soil/groundwater and surface water | inorganic parameters (for all aggregates): Release: antimony (Sb) / arsenic (As) / barium (Ba) / cadmium (Cd) / chromium (Cr) / cobalt (Co) / copper (Cu) / mercury (Hg) / lead (Pb) / molybdenum (Mo) / nickel (Ni) / selenium (Se) / tin (Sn) / vanadium (V) / zinc (Zn)/ bromide (Br) / chloride (Cl) / fluoride (F) / sulphate (SO4) further substances (for recycled and relevant manufactured aggregates) Release: pH / electrical conductivity / chromate / ammonium-N / nitrite-N / cyanide / AOX / DOC / PAH / hydrocarbons / phenol Content: EOX / TOC / arsenic / lead / cadmium / chromium / copper / zinc / benzene / ethylbenzene / toluene / xylenes (sum, being the sum of m-xylene, p-xylene and o-xylene) / phenol / naphthalene / phenanthrene / anthracene / fluoranthene / chrysene / benzo(a)anthracene / benzo(a)pyrene / benzo(k)fluoranthene / indeno(1,2,3cd)pyrene / benzo(ghi)perylene / PAHs (sum of the afore mentioned PAH and EPA PAH) / PCBs (sum of regulated congeners) / mineral oil / asbestos (weighed, serpentine asbestos plus amphibole asbestos) | 2006-557-NL, 2004-71-D, 2005-735-FIN, 2006-223-E, 2007-653-A |
|  |  | bound use in road construction | recycled aggregates and manufactured aggregates Release: pH / electrical conductivity / ammonium-N / nitrite-N / chloride (Cl-) / sulphate (SO4-) / cyanide / fluoride (F-) / AOX / DOC / PAH (EPA) / phenol index / antimony (Sb) / arsenic (As) / barium (Ba) / lead (Pb) / cadmium (Cd) / chromium (Cr) / chromate / copper (Cu) / molybdenum (Mo) / nickel (Ni) / mercury (Hg) / selenium (Se) / vanadium (V) / zinc (Zn) / mineral oil hydrocarbons / PCB Content: EOX / TOC / arsenic / lead / cadmium / chromium / copper, zinc / PAH / PCB / hydrocarbons | 2004-71-D, 2005-735-FIN, 2006-223-E, 2007-653-A |
| EN 13450 | Aggregates for railway ballast | unbound use of any/all aggregates in railway construction | inorganic parameters (for all aggregates) Release: antimony (Sb) / arsenic (As) / barium (Ba) / cadmium (Cd) / chromium (Cr) / cobalt (Co) / copper (Cu) / mercury (Hg) / lead (Pb) / molybdenum (Mo) / nickel (Ni) / selenium (Se) / tin (Sn) / vanadium (V) / zinc (Zn) / bromide (Br) / chloride (Cl) / fluoride (F) / sulphate (SO4) further substances (for recycled and relevant manufactured aggregates): Release: pH / electrical conductivity Content: benzene / ethylbenzene / toluene / xylenes (sum, being the sum of m-xylene, p-xylene and o-xylene) / phenol / naphthalene / phenanthrene / anthracene / fluoranthene / chrysene / benzo(a)anthracene / benzo(a)pyrene / benzo(k)fluoranthene / indeno (1,2,3cd) pyrene / benzo(ghi)perylene / PAHs (sum of the afore mentioned PAHs) / PCBs (sum of PCB 28, 52, 101, 118, 138, 153 and 180) / mineral oil /asbestos (weighed, serpentine asbestos plus amphibole asbestos) Sensory evaluation: organoleptic test | 2006-557-NL, 2006-223-E, 2006-516-D |

| EN Number | Standard Title | Regulated field(s) of application | Requirements | Notified regulations. |
| --- | --- | --- | --- | --- |
| EN 13383-1 | Armourstone - Part 1: Specification | Use in protective structures in water construction | inorganic parameters (for all aggregates) Release: antimony (Sb) / arsenic (As) / barium (Ba) / cadmium (Cd) / chromium (Cr) / cobalt (Co) / copper (Cu) / mercury (Hg) / lead (Pb) / molybdenum (Mo) / nickel (Ni) / selenium (Se) / tin (Sn) / vanadium (V) / zinc (Zn)/ bromide (Br) / chloride (Cl) / fluoride (F) / sulphate (SO4) further substances (for recycled and relevant manufactured aggregates) Content: benzene / ethylbenzene / toluene / xylenes (sum, being the sum of m-xylene, p-xylene and o-xylene) / phenol / naphthalene / phenanthrene / anthracene / fluoranthene / chrysene / benzo(a)anthracene / benzo(a)pyrene / benzo(k)fluoranthene / indeno (1,2,3cd) pyrene / benzo(ghi)perylene / PAHs (sum of the afore mentioned PAHs) / PCBs (sum of PCB 28, 52, 101, 118, 138, 153 and 180) / mineral oil /asbestos (weighed, serpentine asbestos plus amphibole asbestos) For steel and metal slags additionally: pH, electric conductivity | 1996-443-D, 2006-557-NL |

NOTE: In regard to Essential Requirement 3 the requirements for information deriving from notified national regulations with respect to relevant substances identified on the EC "Indicative List" have to be fulfilled for relevant aggregates/aggregate categories when and where construction works in which they are used are subject to national regulations containing such requirements. Such harmonisation requires that provisions for the relevant regulated substances have to be included in the affected aggregate standards in an appropriate manner. Where the aggregate/aggregate type is to be used in construction works not subject to a relevant notified national regulation for a substance, performance need not be determined. If additional substances are regulated for the products within the scope of M/125, they may be added to this mandate. This applies both to notified regulations that may have been overlooked when preparing this amendment, or to any new notified regulations.