REGULATORY QUERIES - December 2012

QUERIES CONCERNING ALREADY CITED hENs

| EN TITLE | QUERIES | CEN / EC / MS POSITION | WAY FORWARD |
| --- | --- | --- | --- |
| EN 438-7 Highpressure decorative laminates (HPL) – Sheets based on thermosetting resins (usually called laminates) | GERMANY: In Germany requirements exist to verify the behaviour of products in respect of continuous glowing combustion in the case of fire. In the case of not considering this behaviour, an additional national assessment e. g. on the basis of national procedures would have to be required. The product standard does not consider ER 3 "Hygiene, health and the environment". Germany has regulations with respect to that product concerning the indoor environment |  | Explanations awaited from DE on how the product is assessed |
| EN 858-1 Separator systems for light liquids - Part 1: Principles of product design, performance and testing, marking and quality control | GERMANY: There are no provisions concerning the mechanical resistance and stability of the separator systems as well as of their tightness against light liquids and regarding the reaction to fire behaviour of components. This has been agreed on within the TC and, as far as we know, with the Commission Services at the time of drafting this standard, because no possibility has been found up to now to overcome this situation with commonly agreed assessment criteria. Nevertheless, it leads to the necessity of verification of additional national requirements which is to be mentioned here. | CEN position: Technical issue to be discussed at TC level. EC position It is in the ess. characteristics in the mandate | TC to investigate |
| EN 1504-3:2005 - Products and systems for the protection and repair of concrete structures - Definitions, requirements, quality control and evaluation of conformity - Part 3: Structural and non-structural repair | AUSTRIA: The following two characteristics are missing, although both characteristics are included in the mandate M/128: 1 Thermal cycling without de-icing salt impact 2 Free shrinkage Thermal cycling without de-icing salt impact: The method of verification is already standardised on European level. EN 12617-4, 05.2002 Products and systems for the protection and repair of concrete structures - Test methods - Part 4: Determination of shrinkage and expansion Both characteristics were subject to Austrian regulations concerning concrete repair products. These regulations were firstly notified under the notification number 1999/248/A concerning the Building Material List ÖA. Further amendments and updates have been notified the same way. Within this list the technical specification you will find under item 3.5.4 Special mortars, Guideline of the Austrian Concrete Association for the preservation and repair of structures built from concrete and reinforced Concrete. This technical specification includes both characteristics. | CEN Position (2011-12-01): Answer from CEN/TC 104/SC8: The standards listed are currently under revision by respective WGs and the queries made will be taken into during the revision process. Therefore, detailed answers to the queries will be formulated by working groups concerned as soon as possible |  |
| EN 1806 Chimneys - Clay/ceramic flue blocks for single wall chimneys - Requirements and test methods | GERMANY: A request to amend M105 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 1825-1 Grease separators – Part 1: Principles of design, performance and testing, marking and quality control | GERMANY: There are no provisions concerning the mechanical resistance and stability of the grease separators as well as of their tightness against grease and regarding the reaction to fire behaviour of components. This has been agreed on within the TC and, as far as we know, with the Commission Services at the time of drafting this standard, because no possibility has been found up to now to overcome this situation with commonly agreed assessment criteria. Nevertheless, it leads to the necessity of verification of additional national requirements which is to be mentioned here. | CEN position: Technical issue to be discussed at TC level. EC position It is in the ess. characteristics in the mandate |  |
| EN 1856-1 Chimneys - Requirements for metal chimneys - Part 1: System chimney products | GERMANY: A request to amend M105 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 1916:2002 + AC:2006 Concrete pipes and fittings, unreinforced, steel fibre and reinforced | FINLAND Finnish authorities and experts have found many deficiencies in EN 1916:2002 + AC:2006 Concrete pipes and fittings, unreinforced, steel fibre and reinforced. We ask the Commission to start actions so that these deficiencies can be corrected as soon as possible. EN 1916 is under revision in CEN TC 165 but unfortunately the latest draft does not include solutions on these problems. There seems to be similar deficiencies in EN 1917 Concrete manholes and inspection chambers, unreinforced, steel fibre and reinforced standard which shall be also corrected. The deficiencies found are the following: Foreword: 7th paragraph: It is not possible to open internal market for construction products if the situation as described in the paragraph exists in the harmonised product standard. "When the text of this European Standard was approved, complete agreement could not be achieved for all requirements in the existing national specifications of CEN members and so it includes only those requirements and associated test methods for which a consensus could be reached. Consensus was achieved on the requirements for quality control.” Standard has to be improved and the above text shall be deleted. CEN TC has the task to agree on the common European verification methods for all harmonised characteristics given in the mandate based on Member States regulations. It is not acceptable that the standard writes do not develop verification methods for all harmonised characteristics. According to the information we have received other national marks or approvals are required in addition to the CE-marking. These products are not necessary regulated by the Member States. Use of these products are under rules and conditions imposed by public bodies or private bodies acting as a public undertaking or acting as a public body on the basis of a monopoly. See article 6(1) of the CPD and also article 8(5) of the CPR. |  |  |
| EN 12101-1 Smoke and heat control systems – Part 1: Specification for smoke barriers | GERMANY: A request to amend M109 has been sent to the Commission Services. "Reaction to fire" and "Durability" should be added for these products. | CEN Position (2011-12-06): Regulatory: EC to amend mandate | Explanations awaited from DE on how the product is assessed. If necessary EC to propose a revised mandate to the SCC. |
| EN 12101 – 2 Smoke and heat control systems – Part 2: Specification for natural smoke and heat exhaust ventilators | GERMANY: 1) A request to amend M109 has been sent to the Commission Services. "Reaction to fire" and "Durability" should be added for these products. 2) Unclear and/or insufficient provisions concerning the initial type-testing after change of a product. German answer to CEN position: Reference is made to the E-Mail of the Federal Ministry on Traffic, Construction and Urban Development to the EC Services from 13 May 2011 where reference has been made with regard to the same issue to former correspondence (Mail from 30 September 2009 from DIBt (Mr Springborn) addressed to the Commission Services (Mr Mikkeli, Mr Katsarakis) as well as to CEN (Mr da Costa), PG 221 (letter from the German Federal Ministry, Mr Guenther) | CEN Position (2011-12-06): Regulatory: EC to amend mandate | Explanations awaited from DE on how the product is assessed. If necessary EC to propose a revised mandate to the SCC. |
| 12101-3 Smoke and heat control systems – Part 3: Specification for powered smoke and heat exhaust ventilators | GERMANY: A request to amend M109 has been sent to the Commission Services. Reaction to fire" should be added for these products. The functioning of the ventilators is only made sure if they are operated in an arrangement as they were tested. The classification of the fire resistance of the ventilators should consider the intended and tested position of the ventilators; the information should be given in the CE marking. Many test results obtained around Europe show that a motor tested in accordance with Annex D (test of the motor without the fan) do not pass the test together with the ventilator. Therefore, Annex D should be deleted. | CEN Position (2011-12-06): Regulatory: EC to amend mandate | Explanations awaited from DE on how the product is assessed. If necessary EC to propose a revised mandate to the SCC. |
| EN 12101-7 Smoke and heat control systems - Part 7: Smoke control ducts | GERMANY: "Reaction to fire" and "Durability" should be added for these products. The standard should be amended accordingly. |  |  |
| EN 12101-8 Smoke and heat control systems - Part 8: Specification for smoke control dampers | GERMANY: "Reaction to fire" and "Durability" should be added for these products. The standard should be amended accordingly. |  |  |
| EN 12326-1 Slates and stone products for discontinuous roofing and cladding – Part 1: product specification | EC services, DENMARK, SWEDEN: The standard covers only slates of tectonic origin. Products placed today legally in the markets of MS are excluded without justification. The standard needs to be revised to include also the above mentioned slates. | CEN Position (2011-12-06): prEN 12326-1, revised to take into account the Brazilian slates, is out on UAP procedure closing on 2012-02-29. The contents of the draft are acceptable to the Brazilian experts. CEN/TC 128/SC 8 is working on a new standard covering the Nordic slates. | TC results expected |
| EN 12566-1:2001. Small wastewater treatment systems for up to 50 PT – Part 1: Prefabricated septic tanks | DENMARK: Concerning these standards we have a number of problems. The problems do not refer to the mandate but to the differences between the voluntary part and the mandatory part of the standard. Annex ZA describes the mandatory part needed for fulfilling the performance characteristics. The effectiveness of treatment also depends of a number of other clauses in the standard. Therefore we suggest to include at least 3 other clauses from the standard in table ZA.1. Clause 5.1.1 describes the inlet and outlet based upon the size of the capacity of the system. Clause 5.6 describes the inlet and outlet pipe arrangements to ensure that no surcharging or back-flow in the inlet pipe occurs at maximum flow rate. From clause 5.7 we need to have the requirements to the access to the outlet and possibility of removing sludge and cleaning and maintenance, and the dimensions of the extension shafts. In notification number 2010/486/DK Draft executive order amending executive order no. 1448 of 11 December 2007 on waste water licences, etc. under Chapters 3 and 4 of the Environmental Protection Act you can find the new Danish requirements. As the amendments do not require changes in the mandate we have also chosen to go further via CEN’s internal procedures with the proposal |  |  |
| EN 12566-3:2005, Small wastewater treatment systems for up to 50 PT – Part 3: Packaged and/or site assembled domestic wastewater treatment plants. | EC services: Table ZA.1 As regards the “water tightness test” there are in fact two tests with one pass level each and two other tests with a choice of either three levels of vacuum in one case and three levels of pressure in the other (Annex A). Ultimately there are eight “pass” options. This contradicts mandate which specifies that one test should be chosen. In fact according to the table A.1, the water test in A.2 is applicable to all the products made of different materials, therefore it is difficult to understand why this test was not chosen in isolation. Similarly for the “crushing resistance and maximum load deformation”, Annex C, table C.1 lists about nine methods to determine structural behaviour whereas only one Method C.6, the “Pit test” is applicable to all materials. In addition all the other methods, other than method C.6, only produce figures to be used in national calculation procedures whilst the method C.6 determines a pass/fail requirement. Again it is difficult to understand why method C.6 was not chosen. Clause ZA.2.1 It contains an incorrect reference to the AoC Decision. It should be 1997/464/EC of the OJ 198, 25-7-1997. Table ZA.3 Assignment of evaluation of conformity tasks reflects the problems identified in Table ZA.1 above. | 2011-02-16: CEN/TC 165 has been asked to give priority to the development of the amendment required to take into account the various comments received from EC, FI ,DE, DA, IE | CEN to provide a roadmap for amend mends in the hEN before the SCC. |
|  | FINLAND: 1) Detailed technical comments included in the previous query document to be dealt by the TC. Annex ZA: Clause ZA.1 The following proxy characteristics shall be added to Table ZA.1 (In addition the answer to the mandate shall be modified accordingly): Under mandated characteristic ´Effectiveness of treatment’: - Hydraulic design, reference to clause 6.1.2 - Access, reference to clause 6.1.3 Clause ZA.3 The following modifications are needed in item e) e) information on the relevant essential characteristics listed in Table ZA.1, as follows: - effectiveness of treatment expressed as mean treatment efficiency ratio of COD, BOD, SS, nitrogen parameters and total phosphorus obtained during nominal sequences together with the tested organic daily load (BODx) with used power consumption and the mean temperature (liquid phase) of the wastewater in the test, - nominal designation … - water-tightness … - load-bearing capacity with the method used and maximum load deformation (as characteristic load bearing capacity) with dry, wet conditions, characteristic pedestrian load and the maximum embedding height and used Nationally Determined Parameters NDPs when relevant, - durability… |  |  |
|  | GERMANY: The product standard does not consider ER 2 "Safety in case of fire". Germany has regulations requesting the determination of the reaction to fire characteristic of that product.. IRELAND: In Ireland, there is a general requirement that on-site domestic wastewater treatment achieves minimum performance standards for BOD, SS and NH4-N which should be 20-30-20 mg/l respectively. In nutrient sensitive areas, a local authority might set more stringent performance standards for NH4-N and, in addition to these 3 parameters, require other parameters such as Total Nitrogen as N and Total Phosphorus to have performance standards of 5 and 2mg/l respectively. Total electrical consumption should also be declared in order that the energy efficiency of the plant can be considered when deciding the overall suitability of the system. All these parameters are referenced and test methods given in EN 12566-3, in fact section 6.3 which deals with Treatment Efficiency states “Where required, i.e. by national regulations, parameters described in B.2.4 shall be declared.” Annex B deals with the Treatment Efficiency test procedure and B.2.4 lists all data that “shall” and “may” be monitored. This list covers all the parameters above, by contrast only COD, BOD and SS together with the tested organic daily load are relevant characteristics in terms of treatment efficiency in Annex ZA.1 Ireland believes that the performance standards for these parameters are absolutely essential in order to protect the environment, prevent pollution of receiving waters. | EC submitted amendment of M118 | For the TC to take mandate and comments into account |
| EN 12566-4:2001 Small wastewater treatment systems for up to 50 PT – Part 4: Septic tanks assembled in situ from prefabricated kits | DENMARK: Concerning these standards we have a number of problems. The problems do not refer to the mandate but to the differences between the voluntary part and the mandatory part of the standard. Annex ZA describes the mandatory part needed for fulfilling the performance characteristics. The effectiveness of treatment also depends of a number of other clauses in the standard. Therefore we suggest to include at least 3 other clauses from the standard in table ZA.1. Clause 5.1.1 describes the inlet and outlet based upon the size of the capacity of the system. Clause 5.6 describes the inlet and outlet pipe arrangements to ensure that no surcharging or back-flow in the inlet pipe occurs at maximum flow rate. From clause 5.7 we need to have the requirements to the access to the outlet and possibility of removing sludge and cleaning and maintenance, and the dimensions of the extension shafts. In notification number 2010/486/DK Draft executive order amending executive order no. 1448 of 11 December 2007 on waste water licences, etc. under Chapters 3 and 4 of the Environmental Protection Act you can find the new Danish requirements. As the amendments do not require changes in the mandate we have also chosen to go further via CEN’s internal procedures with the proposal |  |  |
| EN 13063-2 Chimneys - System chimneys with clay/ceramic flue liners - Part 2: Requirements and test methods under wet conditions | GERMANY: A request to amend M105 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 13069 Chimneys - Clay/ceramic outer walls for system chimneys - Requirements and test methods | GERMANY: A request to amend M105 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 13162 “Thermal insulation products for buildings - Factory made mineral wool (MW) products – Specification” to EN 13171 | GERMANY: For the use as components of load-bearing construction products or systems information on characteristic values for materials are missing (presently only giving of mean values) as well as on the apparent density (for design loads and normally for assessing the reaction to fire). Instead of a nominal value for the thermal conductivity (90/90 value) the indication of a limit value is needed. This limit value may not be exceeded by any part of the production. There are no provisions given for sampling and installation in the test room in the context of fire classification. In Germany requirements exist to verify the behaviour of products in respect of continuous glowing combustion in the case of fire. In the case of not considering this behaviour, an additional national assessment e.g. on the basis of national procedures would have to be required. | CEN position Several national test methods on glowing combustion available. CEN in difficulty to adopt one EC position: The TC needs to take into account the opinion of the NSO of the MS which regulate on the specific additional characteristic (unless this would lead to evident discriminations). |  |
| EN 13240 Room heaters fired by solid fuel | FINLAND: Problems which are related to a not appropriate test method and by not taking into account thicker insulation layers necessary in very cold climates |  | TC expected to provide a roadmap and a timetable for the changes |
| EN 13241 Industrial, commercial and garage doors and gates - Product standard - Part 1: Products without fire resistance or smoke control characteristics | GERMANY: A request to amend M101 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 13492 Geosynthetic barriers – Characteristics required for use in the construction of liquid waste disposal sites, transfer stations or secondary containment | GERMANY: The product standard does not consider ER 2 "Safety in case of fire". Germany has regulations requesting the determination of the reaction to fire characteristic of that product. |  |  |
| EN 13747:2008 “Precast concrete products - Floor plates for floor systems” | FINLAND: Detailed remarks on EN 13747/prA1 ZA.3.2 (Method 1) No guidance is given how to declare the harmonised characteristic durability. The basic idea of the CE-marking to declare the harmonised characteristic in a comparable way between different manufacturers is not achieved. ZA.3.3 (Method 2) Declarations of serviceability limit state aspects (cracking and deflection) are missing from Figure ZA.3. This shall be part of the harmonised characteristic mechanical strength as explained in the Annex 1 of the CPD. In Finland we have serviceability limit state requirements in our regulations. No guidance is given how to declare the harmonised characteristics geometrical data, detailing, durability, acoustic insulation parameters, complementary information on fire resistance and other NDPs. The basic idea of the CE-marking to declare the harmonised characteristic in a comparable way between different manufacturers is not achieved. Term ‘Position number’ in Figure ZA.3 should be clarified. In Method 2 it cannot mean the location of the precast concrete unit in the works. ZA.3.4 (Method 3a) These are so called made to measure products produced according to a specific client’s order and where structural design of the product is outside the CE-marking. Structural design is performed by the purchaser or by the manufacturer in cooperation with the purchaser. When fixing the CE-marking on the product the manufacturer just declares that he has produced the product according to the production drawing (e.g. used the materials specified in the production drawing). Structural design can be performed according to any (national) design code but it is not part of the CE-marking. Clear reference to the production drawings (number and date) are missing in Figure ZA.4. Reference to the client’s order is also needed if it is not given in the production drawing. Figure ZA.4 can be simplified since there is no need to declare geometrical data and detailing in the CE-marking since they are already given in the production drawing. In addition the manufacturer cannot declare mechanical strength, fire resistance, acoustic insulation parameters and durability in the CE-marking since in this method those characteristic shall be considered outside the CE-marking e.g. by the client. ZA.3.5 (Method 3b) These are so called made to measure products which are produced for a specific works and where structural design of the product is part of the CE-marking. We cannot accept that "This method applies also in the case of a structural design made with means other than EN Eurocodes". This is an important safety issue. Our arguments are the following: Generally mandated characteristics can be declared based on testing or calculations as specified in the relevant harmonised product standard (hEN). In hENs reference is made to the European test methods given in EN-standards. It is not acceptable to write in hENs that testing can be made by means other than by test xxx determined in standard yyy. In the CPD there is nothing which gives a right to treat calculations differently than testing. The tasks and duties of notified bodies have been mentioned in Guidance Paper K and Guidance Paper A. Notified bodies are recognized to be competent by Member States. The competence is based on the CPD, Guidance Papers and relevant hENs. Notified bodies have to understand required tests and calculations rules. Building authorities cannot demand anything else as mentioned in CE-marking. Finnish building authorities cannot accept imported construction products for structural use which have been calculated according to national design rules as part of the CE-marking. Criteria for the notified bodies verifying structural design calculations of the products do not include the knowledge on the national design codes of all 27 Member States. But they include the knowledge on the Eurocodes. Method 1 and 3a are for those cases where national design codes are used but then calculations are outside the CE-marking. These calculations are treated and checked according to national practices valid in the country where the product will be used. No guidance is given how to declare the harmonised characteristics geometrical data, detailing, mechanical resistance, fire resistance, acoustic insulation parameters and durability. The basic idea of the CE-marking to declare the harmonised characteristic in a comparable way between different manufacturers is not achieved. In Figure ZA.5 reference to the client’s order shall be replaced by specifying the works and the position number of the product in the works. 3. General remarks Attestation of conformity procedures for structural construction products It is up to CEN TC to decide which methods of Guidance Paper L are to be introduced in each hEN for structural products. Sometimes only method 1 is valid e.g. for sawn timber. Normally it will be useful to have all methods since it is difficult to know which methods may be needed in the future. It is important to understand that the attestation of conformity tasks is different for different GP L methods. These tasks have been described in hENs in Table ZA.3 –Assignment of evaluation conformity tasks. This table is difficult to understand and do not give detailed information to the manufacturers and the notified bodies. I think that it would be more useful to have separate tables for each GP L method. Harmonised product standards shall describe more in detail the tasks of the manufacturers when performing the structural design calculations and the tasks of the notified bodies when verifying these calculations according to Guidance Paper K. I refer to Guidance Paper K where it has been explained that calculations can be a part of Initial Type Calculations (long series) or "calculations performed on individual manufactured product may be part of the factory production control in analogy to testing of samples taken at the factory" (small series). Method 2 is usually for long series products where production can continue years based on the same calculations. Then it is wise that notified body "checks" these calculations. Methods 3a and 3b relate to small series production. It is impossible that the notified body will "check" all calculations which have been carried out by the manufacturer. But the notified body can "check" some calculations in analogy to testing of samples at the factory. In the case of small series "checking of calculations" can be also a task of the inspection body. I refer to the Annex III of the CPD "inspection body, …. such as assessing, recommending for acceptance and subsequent audit of manufacturers' quality control operations, and selection and evaluation of products on site or in factories or elsewhere, according to specific criteria;" The problem is that the hENs for precast concrete products do not give detailed instructions how to carry out the tasks and duties which relate to calculations mentioned in Guidance Paper K. In many cases GP K states "in accordance with the provisions of the hEN". But in many cases there are no provisions or only very general provisions in the hENs. We think that this is not acceptable. It shall be described in the same detailed way as for testing. We have to be able to rely the manufacturers and notified bodies carry out these tasks and duties in a coherent way. These tasks and duties cannot vary very much from one Member State to another. It is credibility issue for the CE-marking and it is also a safety issue. Service limit state requirements In Finland we have service limit state requirements in our regulations. These service limit state requirements are not in Annex ZA. These shall be added. 4. Long term objectives (valid for all harmonized product standards for structural products) 4.1. User friendliness of standards and coherence of harmonised standards for structural use Harmonised product standards refer to technical information, technical documentation, design specification etc. The content of these terms have not been defined coherently between harmonised product standards for structural use. In Annex M of EN 13369 there are some explanations. Harmonised product standards should be such that these are easy to understand and these can be understood in the same way. The content of these definitions shall be determined more in detail. 4.2. Attestation of conformity for calculations Attestation of conformity decisions are based on article 13.4 of the CPD. There is nothing in the article which relates to the risks related to the calculations (for structural use). There are attestation of conformity decisions 1; 2+ and 3 (seldom) for products for structural use. But the risks related to the calculations may be the same. In some cases we can conclude that the risks related to the calculations are less for some AoC 1 products than for some AoC 2+ products. The CPD shall treat fairly manufacturers. The requirements shall be based on objective reasons. It would be useful to have additional criteria for the risks related to the calculations and to determine the attestation of conformity systems for calculations in some new way. | The position of CEN/TC 229 of 2009-06-17 has not changed. Awaiting EC letter 2009-06-17 CEN/TC 229 Chairman: I am very happy to see that M.J.Virtanen recognizes that new Annex ZA (that we generalize) provides a great improvement. Now he sends us new comments, I can understand some of them as general, others are the result of M.J.Virtanen's personal interpretation. I will try to answer to each comment by reference to our last model of Annex ZA. - ZA.3.2 (Method 1): in ZA 3.2 it is written "provisions for durability", I recognize that it is not a guidance but "durability" is presently a NDP so it is was not possible to be more precise. When common rules will exist on that item, we will apply them. -ZA.3.3 (Method 2): Serviceability limit state aspects are mandatory in Finland, but not in all Member States! So they are NDPs and reference to NDPs is made in the text. I agree that it is not given any common declaration method for the items listed by Matti. The term "position number" is probably not exactly the best. -ZA.3.4 (Method 3a): reference to design specifications is mentioned in Figure ZA.4 and traceability is covered by FPC. I do not understand how figure Z.A.4 can be simplified as the characteristics mentioned there are explicitly required by the Mandate. About the addition mentioned by Matti, it does not cover items declared by the producer, it is only an information to indicate where they can be found. -ZA 3.5 (Method 3b): Use of other design methods than Eurocodes is explicitly allowed (as a transitional possibility) by GP L 3.3.4. Moreover, as far as I know, criteria for Notified Bodies do not include the knowledge of Eurocodes and for AoC system 2+ it not their task to check the calculations: their task is (for method 3b) to check whether the calculation is covered by FPC (including Initial Type Calculation) and documented or not (we had long discussions in SG13 about that point). Reference to client's order is fundamental, works and position number are part of the product traceability and covered by FPC. 3 General remarks: 3.1: attestation of conformity procedures: I already answered. For Aoc 2+, the NB has only to check that calculations under M2 and 3b are covered by FPC. 3.2: service limit state: it is a Finnish national requirement. 4 Long term objectives: In CEN/TC229 we try to improve continuously our standards. Now we are revising Common Rules (EN 13369). About calculations, the topic is not so simple: the responsibility of the producer should be the same for calculation as for tests. | EC to send a letter to CEN reminding the TC to consider durability aspects |
| EN 13813 Screed material and floor screeds – Screed materials – Properties and requirements | GERMANY: The product standard does not consider ER 3 "Hygiene, health and the environment". Germany has regulations with respect to that product concerning the indoor environment. |  |  |
| EN 13950 Gypsum plasterboard thermal/acoustic insulation composite panels - Definitions, requirements and test methods | GERMANY: In Germany requirements exist to verify the behaviour of products in respect of continuous glowing combustion in the case of fire. In the case of not considering this behaviour, an additional national assessment e. g. on the basis of national procedures would have to be required. |  | Explanations awaited from DE on how the product is assessed |
| EN 13964 Suspended ceilings | GERMANY: In Germany requirements exist to verify the behaviour of products in respect of continuous glowing combustion in the case of fire. In the case of not considering this behaviour, an additional national assessment e. g. on the basis of national procedures would have to be required. | CEN Position (2011-12-06): We cannot remember having been informed that there was an outstanding query regarding EN 13964. I suggest that this issue may be taken aboard in the on-going work. It seems to be simply a matter of introducing the right model clauses in the standard | Explanations awaited from DE on how the product is assessed |
| EN 13986 Wood-based panels for use in construction | GERMANY: In Germany requirements exist to verify the behaviour of products in respect of continuous glowing combustion in the case of fire. In the case of not considering this behaviour, an additional national assessment e. g. on the basis of national procedures would have to be required. GERMANY: In Germany requirements exist for the emission of formaldehyde from wood-based panels and for formaldehyde concentrations in indoor air in buildings. The standard contains test methods which do not ensure that the limit values for indoor air are met. The cited reference test method uses an air exchange rate that is unrealistically high for modern buildings and a loading factor that underestimates the emission when also other surfaces than walls are covered with wood-based panels. Germany requires a change of the ventilation rate from 1.0 per hour to 0.5 per hour and higher loading factors for products that are foreseen for multiple surfaces (floors, ceilings, walls). | CEN Position (2011-12-06): Answer from CEN/TC 112: Glowing combustion: For the determination of glowing combustion a general European test method should be developed. For wood-based panels there is no particular need for action. Mounting and fixing: It is specified in the harmonized standard for wood-based panels (EN 13986) that the panels shall be mounted and fixed in a manner representative of its intended end use if a determination of the reaction to fire is necessary. For wood-based panels this determination is only necessary for special boards which are outside the CWFT classification. It is not intended to develop a specific standard for these panel types. Representatives of CEN/TC 112 have explained in the EGDS that they are opposed to changing the conditions of their test method due to the expectation that ca. 30 % of the products that currently reach the formaldehyde class E1 would not reach it, if the test conditions are changed. In CEN/TC 351 the representatives of CEN/TC 112 have refused to accept the use of the harmonised horizontal test method of CEN/TC 351 or even of the harmonised test conditions (reference room) for wood-based panels. In the amended draft FprEN 13986 a voluntary and stricter formaldehyde class E1plus is proposed. The proposed E1plus class is not sufficient to cover the changes required by Germany. |  |
| - EN 14080:2005 Timber structures – Glued laminated timber – Requirements | EC services: A couple of mandated characteristics are missing from the Table Z.1. The first is dimensional stability although there is a number of long-term loading and creep tests in the body of the document which would probably suffice if referenced. The second omission is the complete disregard of the mandated characteristic resistance to fire, there is no reference to it in table Z.1, nor any mention of the subject in the body of the standard. GERMANY: In the standard complete and sufficient provisions concerning the adhesives and the bonding relevant tests procedures are missing. . |  |  |
| EN 14188-1 Joint fillers and sealants – Part 1: Specifications for hot applied sealants | GERMANY: The product standards do not consider ER 2 "Safety in case of fire". Germany has regulations requesting the determination of the reaction to fire characteristic of that product. |  | Explanations awaited from DE on how the product is assessed |
| EN 14188-2 Joint fillers and sealants – Part 2: Specifications for cold applied sealants | GERMANY: The product standards do not consider ER 2 "Safety in case of fire". Germany has regulations requesting the determination of the reaction to fire characteristic of that product. |  | Explanations awaited from DE on how the product is assessed |
| 14188-3 Joint fillers and sealants – Part 3: Specifications for preformed joint seals | GERMANY: The product standards do not consider ER 2 "Safety in case of fire". Germany has regulations requesting the determination of the reaction to fire characteristic of that product. |  | Explanations awaited from DE on how the product is assessed |
| EN 14190 Gypsum plasterboard products from reprocessing - Definitions, requirements and test methods | GERMANY: In Germany requirements exist to verify the behaviour of products in respect of contentious glowing combustion in the case of fire. In the case of not considering this behaviour, an additional national assessment e. g. on the basis of national procedures would have to be required. Further German explanation: The standard contains test methods which lead to the calculation of mean values. Nevertheless, these mean values are not reasonable - especially in terms of the economically acceptable use of the products concerned - to be converted into characteristic values by being multiplied with national partial safety factors. Furthermore, the standard does not contain any characteristic value for bending, compression, tension and embedding strength which are essential according to the respective design codes. |  | Explanations awaited from DE on how the product is assessed |
| EN 14342+A1 on wood and parquet floorings - | EC services According to information received the TC is planning to amend the standard in order to clarify that bamboo floorings are excluded. Standards are expected to allow all products to declare their performance. Therefore it is unacceptable to discriminate against bamboo floorings, | CEN reply: Scope of EN 14342: This European Standard defines and specifies for wood and parquet flooring products the relevant characteristics, requirements and appropriate test methods to determine these characteristics when used as internal flooring, including enclosed public transport premises". It also applies to wood veneer floor coverings. This document covers flooring defined in EN 13226, EN 13227, EN 13228, EN 13488, EN 13489, EN 13990, EN 13629, !EN 14761" and EN 14354. It does not specify dimensional limits or tolerances, which are given by these same standards. EN 13226: " Wood flooring - Solid parquet elements with grooves and/or tongues" EN 13227: Wood flooring - Solid lamparquet products EN 13228: Wood flooring - Solid wood overlay flooring elements including blocks with an interlocking system EN 13488: Wood flooring - Mosaic parquet elements EN 13489: Wood flooring - Multi-layer parquet elements EN 13990: Wood flooring - Solid softwood floor boards EN 13629: Wood flooring - Solid pre-assembled hardwood board EN 14761 " Wood flooring - Solid wood parquet - Vertical finger, wide finger and module brick" | CEN TC expected to amend the standard not to exclude bamboo floorings |
| EN 14351-1 Windows and doors - Product standard, performance characteristics - Part 1: Windows and external pedestrian doorsets without resistance to fire and/or smoke leakage characteristics | GERMANY: A request to amend M101 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  | EC position: Only components need to be assessed |
| EN 14471 Chimneys - System chimneys with plastic flue liners - Requirements and test methods | GERMANY: A request to amend M105 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 14411:2006 “Ceramic tiles - Definitions, classification, characteristics and marking” | POLAND: l. A reference to the standard EN 14411:2 006 on ceramic tiles has been published in OJ on the 4 December 2007. This standard is replacing the previous one, i.e. EN 14411 :2003 and in the Communication of the Commission there is a note in the last column stating that validity of the EN 14411:2 003 has expired on 30.06.2007. It means that there was no basis for the manufacturers to affix CE marking on these products in the period from 1 July 2007 up to the 4 December 2007. This lack of continuity of the reference harmonized standard available poses a serious problem to the producers of those products with placing them on the market, since market surveillance authorities, acting in accordance with the regulations, deny legality of such CE marking and refuse to accept it. 2. Besides, the new scope of the standard E N 14411:2006 has excluded a number of products which were previously covered by the standard EN 14411:2003 (such as edges, corners, skirting, capping, coves, beads, steps, curved tiles and others). Since all ceramic tiles are covered by the mandate M119 (see annex 1) there is a need to decide, whether or not the excluded products are foreseen to be harmonized, however if a new standard will not be prepared for them the products will be excluded from the CPD. | CEN Formal Vote to be closed June 2012. The draft now deletes the exception and allows the products excluded to be covered again. |  |
| EN 14604:2005 Smoke alarm devices | EC services: ZA.1 All the characteristics listed in Mandate M109 are dealt with except for the characteristic "Safe containment of radioactive sources". The reason for ignoring this characteristic is contained in a note to the Scope which says that "The national requirements for radiation protection differ from country to country and they are not specified in this document. Such smoke alarms should however comply with the applicable national requirements". In the spirit of harmonisation and approximation this item should be re-visited. In the second column of table ZA.1, under the heading "Clauses in this European Standard" a number of clause references is given. It is suggested that clauses 4.12 and 4.18 are not relevant in row 1. Similarly clauses 4.1, 4.2, 4.3 and 4.19 are not relevant to row 2. | 2010-10-15 CEN/TC 72 working on a revision of the work programme. | TC 72 expected to amend its reply to the mandate and to send it to EC for approval. |
| EN 14695:2010 - Flexible sheets for waterproofing - Reinforced bitumen sheets for waterproofing of concrete bridge decks and other trafficked areas of concrete - Definitions and characteristics | GERMANY: The product standard does not consider ER 2 "Safety in case of fire". Germany has regulations requesting the determination of the reaction to fire characteristic of that product. |  |  |
| EN 14814:2007 Adhesives for thermoplastic piping systems for fluids under pressure - Specifications | GERMANY: The product standard does not consider ER 2 "Safety in case of fire". Germany has regulations requesting the determination of the reaction to fire characteristic of that product. Long term stability and medium stability of the adhesive are not verified in the context of the standard. Therefore, a safe use for water polluting liquids is not ensured. |  | Explanations awaited from DE on how the product is assessed |
| EN 14963 Roof coverings – Continuous rooflights of plastics with or without upstands – Classification, requirements and test methods | GERMANY: No characteristic values for the mechanical resistance of roof lights without upstands, are therefore applicable in interiors only. The design of construction works with respect to permanent loads (e.g. snow loads) for roof lights with upstand is not possible on the basis of the declared values given on the basis of this standard. The products are, therefore, applicable in snow-free areas only or if it is has been made sure that there is snow only for a short time. German note: As far as we know the revision of the standard in CEN/TC 128 started in February 2012. As soon as the deficiencies are solved within an agreed version of the standard the respective German comments will be withdrawn. |  |  |
| EN 14989-1 Chimneys - Requirements and test methods for metal chimneys and material independent air supply ducts for roomsealed heating applications - Part 1: Vertical air/ flue terminals for C6-type appliances | GERMANY: A request to amend M105 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 14989-2 Chimneys - Requirements and test methods for metal chimneys and material independent air supply - Part 2: Flue and air supply ducts for room sealed appliances | GERMANY: A request to amend M105 has been sent to the Commission Services. "Reaction to fire" should be added for these products. |  |  |
| EN 15250 Slow heat release appliance fired by solid fuel – Requirements and test methods | GERMANY: Indications on necessary product characteristics are missing: reaction to fire, density, open porosity, temperature possible to be applied in use, resistance against changes in temperature, linear elongation due to temperature, dimensions and tolerances, thermal conductivity, specific heat capacity, specific retention capacity. Specific provisions for construction materials for combustion chambers and masses capable of heat storage are missing; all materials – e.g. natural materials – would be possible. No restriction on fuels "wood logs" or "lignite briquettes". Therefore, all fuels were possible, irrespective of the durability of the material of the combustion chambers. No specifications to determine the nominal heat output of the slow heat release appliances. FINLAND: Problems which are related to a not appropriate test method and by not taking into account thicker insulation layers necessary in very cold climates. |  | TC expected to provide a roadmap and a timetable for the changes |
| EN 15274 General purpose adhesives for structural assembly - Requirements and test methods | GERMANY: The standard does not consider ER 2 contrary to the respective mandate. Additional German remarks: Provisions concerning the fire behaviour (reaction to fire) which consider "mounting and fixing" rules have to be included. Clarifications from DE authorities sent 13/05/2011 | 2010-10-15: CEN/TC 193 response to the mandate M/127 justifying why ER2 has not been considered has been accepted by the EC. |  |
| EN 15275 Structural adhesives | GERMANY: The standard does not consider ER 2 contrary to the respective mandate. Clarifications from DE authorities sent 13/05/2011 | 2010-10-15: CEN/TC 193 response to the mandate M/127 justifying why ER2 has not been considered has been accepted by the EC. |  |
| EN 15498:2008 - Precast concrete products - Wood-chip concrete shuttering blocks - Product properties and performance | GERMANY: In Germany requirements exist to verify the behaviour of products in respect of continuous glowing combustion in the case of fire. In the case of not considering this behaviour, an additional national assessment e. g. on the basis of national procedures would have to be required. |  | Explanations awaited from DE on how the product is assessed |
| EN 15821 Sauna stoves | FINLAND: Problems which are related to a not appropriate test method and by not taking into account thicker insulation layers necessary in very cold climates. |  | TC expected to provide a roadmap and a timetable for the changes |

QUERIES CONCERNING CANDIDATE hENs

| EN TITLE | COMMENTS | CEN POSITION | WAY FORWARD |
| --- | --- | --- | --- |
| Candidate hEN on fire resistant doors + windows | EC comments: Serious delays / Meeting with EC services required |  |  |
| prEN 1013: Light transmitting single skin profiled plastics sheeting for internal and external roofing, wall cladding and ceiling finishes - Part 1: General requirements and test methods | FRANCE: The French authorities consider that Standard PrEN1013 should take more account of Mandate 122 (concerning the establishment of harmonised standards for roof coverings, roof lights, roof windows and ancillary products) and particularly of the essential requirements regarding health and safety. Account should be taken of the resistance of roofing sheets to the soft impact corresponding to a person walking normally on the roof and accidentally tripping and falling for a variety of reasons, it being assumed that the sheets were fitted in accordance with the conditions governing their intended use when they were first placed on the market. It should be pointed out that the failure to take account of Mandate 122 and Article 5.1 of the construction products Directive (CPD) has already led the French authorities to make a formal objection concerning safety in the use of fibre-cement profiled sheets in accordance with Standard EN494 of 2004. It would be desirable, therefore, if some thought were given to the organisation conditions with a view to avoiding problems of "non-quality" with PrEN 1013 similar to those encountered with EN494. This was mentioned as long ago as 14 May 2008. The note to Table ZA.1.1 and the addition of an Article 5.8.2. can not be regarded as an adequate response to the seriousness of the situations arising. Note at the end of Table ZA.1.1 : “a Soft body impact resistance depends highly on the method of installation. Soft body impact resistance of a fully installed assembly/kits shall be declared separately by the manufacturer or designer of assembly/kits as stated in 5.8.2 where subject to regulation, otherwise "NPD" may be declared.” 5.8.2 Soft body impact resistance “The method(s) of installation shall consider the resistance to soft body impact. In the absence of a European test method and where subject to regulations, the soft body impact resistance may be assessed separately in accordance with the requirements of ETAG 010 or individual national safety requirements.” Similarly, it should be borne in mind that ETAG 010 refers to Standard EN15057:2006 in connection with the description of these same tests. With this type of material, however, tests with a soft object are not sufficient, since account needs to be taken of the aging of the materials, which very rapidly – in less than five years – lose their resistance to soft impacts. Finally, the question should be examined of how the danger of people falling could be indicated explicitly in the CE marking by means of a specific logo or pictogram. | 2011-02-16: CEN/TC 128 to draft an amendment to original work programme to cover the issue. The mandate already includes the characteristic and does not need to be amended. |  |
| EN 10080 Reinforcing steel | EC comments: Serious delays / Meeting with EC services required Germany: 1. The future EN 10080 should consider the declaration of the characteristic values of tensile and tensile yield strength in a way that for every piece of reinforcing steel the declaration of its characteristics is clearly identifiable, non-reversible and unmistakable. 2. The clearly identifiable declaration without any ambiguity is particularly required since according to the current EN 10080 also reinforcing steel with yield strengths below 400 MPa or above 600 MPa can be CE marked as reinforcing steel but these steel grades are not suitable for constructions designed according to EC2. 3. The non-reversible identification and the retraceability over a longer period of time - both with regard to the most relevant performance characteristics of the reinforcing steel - shall be ensured independently (and not only depending on information possibly subsequently given by the manufacturer). This is required by Mandate M115 rev1, chapter III (10), clause 10. 4. As Germany has been insisting over a long period of time, the clearly identifiable, non-reversible and unmistakable declaration can be reached with predefined levels of strength indicated on every piece of reinforcing steel. 5. Furthermore, it should be mentioned that the majority of steel product standards manage to work with predefined levels of strength (e.g. EN 523, EN 10025, EN 10088, EN 10138, EN 10337, EN 10348). 6. There are no collectives of product properties which would make it possible to reference levels of performance of the works which under the Mandate, chapter I, clause 3 is the right of the Member States. For the same reason, it is not possible to establish which steels comply with the requirements for reinforcement according to Eurocode 2 (c.f. EN 1992-1-1, Annex C) and therefore fail the requirement posed in chapter III clause 4 of the Mandate M115rev1. 7. There are two methods of assessing bond strength, which are not compatible with each other, bond strength is one of the properties to be declared in the CE marking, this falls short of the requirement posed in Chapter II clause 9 of the Mandate M115. Moreover, defining bond strength by surface geometry as in the present draft shall be differentiated for the different possible yield strengths (this is missing in the present draft), otherwise it is so inaccurate as to pose a serious safety problem and would be in breach of the requirement posed in chapter III clause 6 of the Mandate M115rev1. |  |  |
| EN 14891 Liquid applied water impermeable products for use beneath ceramic tiling bonded with adhesives - Requirements, test methods, evaluation of conformity, classification and designation | GERMANY: The product standard does not consider ER 2 "Safety in case of fire". There are German regulations requesting the determination of the reaction to fire characteristic of that product which have necessarily to be considered. The standard does not cover characteristics which are necessary for the function of the installed waterproofing system such as primers, inlays, reinforcement strips and adhesives for ceramic tiles and the watertight function as an installed system with essential details. |  |  |
| EN 15012 Plastics piping systems – Soil and waste discharge systems within the building structure -Performance characteristics for pipes, fittings and their joints | GERMANY: 1. Reference is made to other non-harmonised standards with respect to the description of the products, e. g. the materials, and to other performance characteristics not covered by this standard. However, this reference is not included, neither directly nor indirectly, in Annex ZA which leads to the fact that it is completely undefined which kind of materials are covered by this standard. This is not acceptable with respect to the defined test methods since there are no experiences on their appropriateness for unknown materials. Also there are no provisions concerning the design of the pipes (thickness of the pipe walls, layers, shape, etc.) 2. The test proposal in question for proving the reaction to fire behaviour of the concerning construction products in the SBI apparatus was already discussed by the Fire Regulators Group in 2001 and 2002. The experts expressed controversial views on the issue and were not consistently in favour of the test proposal presented. Please see therefore Fire Regulators Group reports of meetings dated 16./17.10.2001 (RG N252 rev1, section 1.7), 27./28.05.2002 (RG N278 rev1, section iii.) and 23./24.10.2002 (RG N297 rev2, section iii.). From the reports it is apparent that the German representative always disapproved the test proposal. The following issues were criticised: The test assembly as presented can not cover end use applications with two or more pipes side by side. In Germany the legal requirement is to discharge rain water and waste water each in separate pipes so that in many applications at least two adjoining pipes have to be foreseen. The German representative in the Fire Regulators Group did not agree to the Commission Services' conclusion that the test assembly covering most of end use applications was satisfying. Additionally, this conclusion should be applied only on condition that there is consultation with CEN TC 127. To our knowledge, this has not happen yet. Further, it remains to be stated that the test procedure according to EN 13823 has been prepared in order to determine the reaction to fire performance of essentially flat products. The treatment of linear products like for example pipes may require special rules, as also annotated in EN 13823. The provisions for the initial type-testing and for the factory production control are insufficient regarding checking the reaction to fire. | CEN Position (2011-12-06): TC155 has modified the strategy abandoning the umbrella for real hEN 2010-10-15 CEN/TC 155 is currently finalizing the revision of its answer to the mandate M/131, which will clarify the situation and allow the comments of the Netherlands and Germany to be considered as far as possible. | TC to revise its reply to M131 taking into account the TC meeting (Paris 2010). Commission will ask Member States to confirm the ess. characteristics |
| EN 15013 Plastics piping systems – Non-pressure drainage and sewerage systems buried in ground – Performance characteristics for pipes, fittings and their joints | GERMANY: Regarding the definition of materials and possible constructions see EN 15012. Standard refers to EN 15012 in respect of reaction to fire assessment. Therefore, see comment above. | CEN Position (2011-12-06): TC155 has modified the strategy abandoning the umbrella for real hEN |  |
| EN 15014 and EN 15015 | GERMANY: Regarding the definition of materials and possible constructions see EN 15012. Standard refers to EN 15012 in respect of reaction to fire assessment. Therefore, see comment above. NETHERLANDS: The answer of CEN (TC155) is discussed in the Netherlands in the CEN Shadow-commission of NEN (Dutch Normalization Institute) and also between the experts, producers and VEWIN (organization of Water Companies, the party that has problems with the standards, because they are not sufficient to guaranty the Quality of the system for drinking water in the Netherlands). Vewin is deeply committed in reaching hEN’s that will guarantee good quality piping systems, especially since it will not be allowed to set additional requirements in tendering procedures on properties that are covered by the HEN’s after CE-marking will be in place. Please find attached the Vewin position regarding the observations of TG3 on NL comments with respect to prEN 15014 (document dated 2011-11-01). It is noteworthy that the comments and proposed changes from the Netherlands have been brought forward after lengthy and thorough discussions and were endorsed by all participants in the Dutch (shadow) commission. Secondly these proposals were considered to be vital by all parties involved in order to achieve hEN’s that will guarantee good quality piping systems. All parties involved are happy to see that TG3 thoroughly considered all of these comments and gave a number of satisfying suggestions and answers. A number of the comments were acceptable to TG3 and will be incorporated. That’s good news. However, a number of points still remain open for discussion. Therefore I ask you to give attention to the Dutch comments. Also I send this email to Mr. G. Bedotti, because tomorrow there is an important meeting where the standard (comments) will be discussed. TG3 and TC155 repeatedly stated that no comments are acceptable that seem to be outside the ‘answer to the mandate’. This particular position, in the opinion of the parties involved, strongly hinders reaching the common goal of establishing hEN’s that guarantee the quality of plastic piping systems (comparing with the existing quality level). Points that still need to be solved in our opinion: - Test methods for assessing material characteristics have to be specified; - Connections and fittings must be tested as in the vENs; - A number of additional mechanical properties/test methods related to durability must be specified. Please find further and detailed comments and proposed changes on these items in the annex. I hope that the Dutch comments are clear and sufficient to put forward in the discussion which repairs are needed. It would be most helpful If CEN can take this into account before we have another query meeting and we have to discuss this subject. We, as any other, want to solve the problem as quick as possible, because the market is waiting for these standard. With everybodies support we should fix this. | CEN Position (2011-12-06): GB: I had a look into the VEWIN comments and I would react as follow: 1 – None of the standards drafted at present by TC155 deal with products placed on the market to be used to supply water intended for human consumption. All the standards are drafted in the framework of mandate M/131 “Pipes, Tanks and Ancillaries not in contact with water intended for human consumption” and not of M/136 Rev 2 “Construction products in contact with water intended for human consumption”. This limitation is clearly stated in the TC155 revised answer to M/131, endorsed by the TC155 Plenary meeting and submitted to the Commission together with the Consultant’s acceptance letter. Specifically regarding EN 15014, it is clearly stated in the scope of the standard that: This European standard specifies requirements for plastics pipes and fittings for pressure applications for water supply, drainage, sewerage and irrigation, as well as for any other pressure application in which other liquids are conveyed with the exception of water intended for human consumption. It gives the associated test/assessment methods. This standard does not cover adhesives, joint sealings and gaskets. Therefore, I would conclude that the reference to the existing NL legislation for the water companies is not appropriate and it is even not applicable / relevant for products covered by EN 15014. 2- No clear what it is meant. The requirements related to mandated characteristics are specified in the product standard (EN 15014) and not in voluntary standards. A cross reference to supporting standards is given in EN 15014 for the test methods contained in supporting standards, to be applied for verifying all the requirements. 3 – A change of the row material can affect the performances of essential characteristics. This is the reason why in clause 6.2 related to the evaluation of conformity the attention is drawn up to the possible need of repeating some ITT (related to essential characteristics) or TT (related to voluntary characteristics) in case the change of row material. On the other hand, it cannot be ignored the fact that a number of standards (EN or ISO) exist and if the modification of the material properties remains within the provisions of these standards, the systematic repetition of tests related to the material properties does not appear justifiable. 4 – The EN 15014 covers components as defined in family 2 and 4 of M/131 and not Piping kits referred to in family 1. The experts agreed that, according to what manufacturers usually place on the market are components (pipes and ancillaries), but not connections/joints. The jointing of pipes is considered as being an installation operation since it is obtained simply by inserting a pipe into another without any “physical” connector. 5 – The revised answer to the mandate M/131 was discussed for a long time by the TC155 experts (NEN representatives were present) reaching the agreement regarding the durability tests methods considered appropriate for assessing the behaviour of the essential characteristics and not the behaviour of material properties (like is the Melt Flow Rate proposed). Considering that: - the VEWIN made reference to water legislation - the experts reached an agreement based on the consensus regarding the present content of the draft EN 15014; - the technical aspects (if any) should not be an issue for the Queries Group but should be dealt with at the CEN level, I do not see any justified reasons for discussing the issue at the next Queries meeting. | TC to revise its reply to M131 taking into account the recent TC meeting (Paris 2010). Commission will ask Member States to confirm the ess. characteristicS |