EUROPEAN COMMISSION ENTERPRISE AND INDUSTRY DIRECTORATE-GENERAL Sustainable Growth and Europe 2020 Sustainable Industrial Policy and Construction.

CPR 06/07/1

AMENDMENT OF MANDATE M/120 TO CEN/CENELEC

CONCERNING THE EXECUTION OF STANDARDISATION WORK

FOR HARMONIZED STANDARDS ON

STRUCTURAL METALLIC PRODUCTS and ancillaries

The existing mandate M/120 is hereby amended as follows:

1) In Annex 1 of the mandate the following row is added at the end of the existing table:

| FORM | MATERIALS | PRODUCTS FOR CONSIDERATION |
| --- | --- | --- |
| sandwich panels | steel, coated steel organic coated steel, stainless steel, aluminium alloys, copper, thermal insulation materials | metal faced sandwich panels (unprotected or protected against corrosion by coating) for structural uses |

2) In Annex 2 of the mandate the following product family is added after product family IV (structural connectors):

V) METAL FACED SANDWICH PANELS FOR STRUCTURAL USES Made from steel, coated steel, organic coated steel, stainless steel, aluminium alloys, copper which include a core of thermal insulation materials and which are placed on the market unprotected or protected against corrosion by coating.

The essential characteristics to be covered by the harmonized standard are:

| BRCW | PERFORMANCE CHARACTERISTIC | Durability |
| --- | --- | --- |
| 1 | - loadbearing capacity - wind resistance - racking strength - fatigue strength - dimensional tolerances - resistance to fixings |  |
| 2 | - resistance to fire - reaction to fire -external fire performance | Y |
| 3 | -Release of dangerous substances -Emission of radioactivity - water permeability - air permeability |  |
| 4 | - impact resistance - deformation |  |
| 5 | - airborne sound insulation - sound absorption |  |
| 6 | - thermal resistance |  |

Note: 	The applicable AVCP system for these products and intended use to be taken into account in the relevant harmonised standard(s) will be defined in a delegated act according to the provisions of Regulation (EU) 305/2011, Art 60(h).