MEETING BETWEEN THE FRENCH AUTHORITIES AND THE EUROPEAN COMMISSION SERVICES

APPLICATION OF DIRECTIVE 89/106/EEC (Construction Products Directive)

FOR ROAD RESTRAINT SYSTEMS

The French Road Authorities have asked for a meeting, which finally took place on 30.05.2013. The following EC services have participated in the meeting:

- DG Enterprise and Industry

- DG Internal Market and Services

I.  BACKGROUND

The purpose of the meeting was to ask the European Commission's opinion on a number of points that were considered problematic for the implementation of Directive 89/106/EEC and Regulation (EU) No 305/2011 for road restraint systems, and to seek approval for the French authorities' proposals in order to find a solution for these problematic points.

Afterwards the meeting discussed the consequences from the lack of certain harmonised parts of the European standard EN 1317 on restraint systems (transitions, terminals, protection systems for motorcyclists), and the absence of provisions regarding repairs of short sections of damaged restraint systems in connection with the CE marking obligation which is in force since 1 January 2011.

II. GENERAL POINTS

- The EC services highlighted the difficulties encountered in establishing the parts of European standard EN 1317 that had not yet been completed (not harmonised), i.e. part 4 (transitions) and 7 (terminals). The time for the citation of these parts of the standard cannot be precisely foreseen.

2. 	The EC services considered that the problems raised by the French authorities had to be seen within a wider context as regards legislation and standards:

- on the one hand, Member States have the obligation to apply the Public Procurement Directive (2004/18/EC), the Construction Products Directive (89/106/EEC) and now the Construction Products Regulation (EU) No 305/2011 and the recent Regulation on standardisation (EU) No 1025/2012;

- on the other hand, the following principle should apply for the questions raised by France: a distinction should be made between the harmonised part of standard EN 1317 - Part 5 (general conditions for assessing the products) and the other parts of the standard EN 1317 which are not harmonised. It followed that the harmonisation of the general conditions (EN 1317-Part 5) allowed the restraint system (as a whole kit) to be CE marked. In case parts of the standard (here Part 4 and Part 7) were not harmonised, the corresponding products could not be CE marked and could be sold, separately, as products without CE marking.

3. 	The EC services agreed that the general provisions above could be applied to motorcyclist protection systems, transitions and terminals which are not covered by harmonised standards.

More generally, the EC services agreed that in the case of restraint systems:

- it was advisable in the calls for tender to prefer specifying requirements concerning the performance of the requested product rather than simply making reference to a marking, as this performance could best be expressed by using harmonised standards such as EN 1317-5 for restraint systems. Voluntary national marking are subject to the conditions stipulated in Article 8 of Regulation (EU) No 305/2011.

- in any case, the regulatory framework defined in Regulation No 764/2008 was applicable for products not yet covered by harmonised standards, given that compatibility between the components had to be ensured, as did safety. It should be noted that the order of preference for the technical specifications that could be used is set out in the Public Procurement Directive (2004/18/EC, Art 23).

Requirements on the product performance may be included in the regulations which the Member States may also issue when applying the CPR in order to meet the national safety requirements for construction works (they may also stipulate minimum performance levels, depending on the road types).

III. SITUATION REGARDING PARTS USED IN REPAIRS AND SYSTEMS THAT HAVE NOT YET BEEN STANDARDISED (motorcyclist protection systems, transitions and terminals)

The EC services then explained several points of a more technical nature, firstly concerning parts used in repairs and then concerning products that had not yet been standardised.

Parts used in repairs:

In this case, the section needing repair has to be taken into consideration so that a distinction can be made between repairs involving replacing small damaged lengths and repairs involving re-equipping large sections.

The EC services took the following position on this point: if a large section needed to be replaced, it had to be borne in mind that this involved installing an overall kit that is required to bear CE marking (except for not yet harmonised products: e.g. transitions, etc.);  where it was a case of repairing a short section of a barrier (for example, less than 100 m) which could not be replaced by components of a kit covered by CE marking, then the damaged parts could be replaced by identical parts that met the initial specifications.

Products not yet covered by standards:

As regards motorcyclist protection kits, it was pointed out that these are covered by a CEN Technical Specification (TS) which does not allow the CE marking of the system. The CE marking for the whole kit including motorcyclist protection elements was therefore not yet possible. Since the addition of motorcyclist protection systems altered the way the restraint system worked, here too it was advisable to use in the contractual terms of reference, the types of national technical specifications defined by the Public Procurement Directive and to carry out a conformity certification process according to the general rules described above in sections II.2 and II.3 above.

For transitions, the Commission specified that, since CE marking of these products was not yet possible, the same approach as for the motorcyclist protection systems should be envisaged.

For terminals, the systems specified in sections II.2 and II.3 should be applied.

In a last point, the Commission's representatives pointed out that, during the period of coexistence of a harmonised European standard, it is still allowed to use the national standard whilst nevertheless ensuring that it was as close as possible to the draft European standard currently under discussion.